SECTION 1: Identification of the substance or mixture and of the company/undertaking

1.1. Product identification

Product form: 			Dust

Name: 				Carbon dioxide

EC No.:				204-696-9

CAS No: 				124-38-9

REACH registration number:		Listed in Annex IV/V REACH, exempt from registration

Product code: 			000010021714

Gross formula: 			CO2

Exemptions from REACH authorisation

requirement:		 	Exempted from REACH registration

1.2. Relevant identified uses of the substance or mixture and uses advised against

1.2.1. Relevant identified uses

Relevant identified uses:		Industrial and professional use. Perform a risk assessment before use

Consumer use
Test gas/ calibration gas

Purge gas, diluent gas, inerting gas
Food applications

Shield gas for welding processes

Use for the manufacture of electronic/photovoltaic components
Extinguishing agent

Use as a biocide

Treatment of water intended for human consumption

It is the responsibility of the end user to ensure that the product supplied is suitable for its intended use.

Use of the substance or mixture: 		Propellant in aerosol cans

Propellant
Coolant

Balance gas for mixtures
Use as a biocide

Blanketing gas
Carrier gas

Chemical synthesis
Combustion, melting and cutting processes

Cooling applications
Fire extinguishing gas

Freezing of food

Gas packaging of foodstuffs

Freezing, cooling and heat transfer
Inflation systems

Agent for promoting plant growth

Head gas, auxiliary gas in pressure systems

Process gas
Laser gas

Rays

Consumer use
Creative, art and leisure activities

Laboratory use

drinks Application

Purge gas, diluent gas, inerting gas

Solvent and extraction agents

1.2.2. Use advised against

Use advised against: 			No

1.3. Details of the supplier of the safety data sheet

CO2-4U
Roggestraat 12
8972 Krombeke
0032 57 30 98 40
info@CO2-4u.be

1.4. Emergency telephone number

Emergency number: +32 (0) 70 245 245 – Poison Control Center (free 24/7)

SECTION 2: Identification of the hazards

2.1. Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP]

Physical hazards				Gases under pressure: Liquid gas H28O

Full text of H- and EUH-phrases: see section 16

Adverse physicochemical, health and environmental effects

No additional information available

2.2. Label elements

Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Hazard pictograms (CLP):

GHS04

Signal word (CLP):				Warning

Gevarenaanduidingen (CLP): 			H280 - Contains gas under pressure; may explode if heated.

Safety recommendations (CLP) storage		P403 - Store in a well-ventilated place

Additional information:			Suffocating in high concentrations

2.3. Other dangers

Other dangers:	High concentrations of CO2 can adversely affect the blood circulation, even at normal levels of oxygen concentration. leading to unconsciousness and death. Symptoms include headache, nausea and vomiting. Not classified as PBT or vPvB. Asphyxiant in high concentrations. Contact with liquid may cause frostbite. The substance/mixture has no endocrine disrupting properties.

SECTION 3: Composition and information on ingredients

3.1. Substances

| Name | Product identification | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| Carbon dioxide | CAS-Nr: 124-38-9 EG-Nr: 204-696-9 REACH-nr: *1 | 100 | 100 Press. Gas (Liq.), H280 |

Full text of H- and EUH-phrases: see section 16
Contains no components that affect the classification of the product.
*1: Listed in Annex IV/V REACH, exempt from registration

*2: Registration not required: substance is produced or imported < 1t/y

3.2. Mixtures

Not applicable

SECTION 4: First aid measures

4.1. Description of first aid measures

First aid after inhalation: 			Move victim to uncontaminated area and use respiratory protection. Keep victim warm 			and at rest. Call a physician. If breathing stops, administer cardiopulmonary resuscitation

First aid after skin contact: 			In case of frostbite, rinse with water for at least 15 minutes. Apply a sterile dressing. Treat 					as burns. Obtain medical attention

First aid after eye contact: 			Rinse eyes immediately and thoroughly with water for at least 15 minutes

First aid after ingestion: 			Ingestion is not considered likely

4.2. Most important acute and delayed symptoms and effects

Main acute and delayed			Low levels of carbon dioxide lead to headaches and accelerated

symptoms and effects				 May cause suffocation in high concentrations. Due to loss of consciousness, the victim is 					not aware of the suffocation. See section 11

4.3. Indication of any immediate medical attention and special treatment needed

None

SECTION 5: Firefighting measures

5.1. Extinguishing media

Suitable extinguishing agents: 		Water spray or mist. Product is not flammable, use fire control measures appropriate to 	surrounding fire

Unsuitable extinguishing agents: 			Do not use a water jet to extinguish the fire

5.2. Special hazards arising from the substance or mixture

Reactivity in case of fire: 			No reactivity hazard other than described in sub-section below

Specific risks:	 			Exposure to fire may cause container to rupture or explode

Hazardous combustion products: 			None

5.3. Advice for firefighters

Specific methods: 				Coordinate fire control measures to adjacent fires. Exposure to fire or radiant heat may 					rupture pressure container. Cool endangered pressure containers with water jet from safe 					position. Do not discharge contaminated fire extinguishing water into drain. Stop flow of 					product if possible. Use water spray or fog to knock down smoke if possible. Move 					containers away from fire if this can be done without risk.

Special protective equipment for the fire brigade: 	Use self-contained breathing apparatus in confined space. Standard protective clothing 					and equipment (self-contained breathing apparatus) for firefighters. EN 469 : Protective 					clothing for firefighters. EN 659 : Protective gloves for firefighters. Standard EN137 - Self-					contained breathing apparatus

SECTION 6: Measures in case of accidental release of the substance or mixture

6.1. Personal precautions, protective equipment and emergency procedures

6.1.1. For persons other than the emergency services

Emergency procedures:			Act according to local emergency plan. Attempt to stop release. Evacuate area. Provide 					adequate ventilation. Prevent entry into sewers, basements, pits or any place where 					accumulation could be dangerous. Keep upwind. See section 8 of the SDS for more 					information on personal protective equipment.

6.1.2. For the emergency services

Emergency procedures: 			Wear self-contained breathing apparatus unless the atmosphere is proven to be safe. 					Oxygen detectors should be used when quantities of asphyxiating gases may be released. 					See section 5.3 of the SDS for more information.

6.2. Environmental precautions

Try to stop the outflow

6.3. Methods and material for containment and cleaning up

Methods and material for containment and cleaning up: 	Ventilate the room

6.4. Reference to other sections

See also sections 8 and 13

SECTION 7: Handling and storage

7.1. Precautions for safe handling of the substance or mixture

Safe use of the product: 			Tanks containing or having contained flammable or explosive substances must not be 					inerted with liquid carbon dioxide. The formation of solid carbon dioxide particles must be 					avoided. To prevent electrostatic discharges, the circuit must be properly earthed. Be 					aware of the risk of static electricity formation when using CO2 extinguishers. Do not use 					them where a flammable atmosphere may be present. Good industrial hygiene and safety 					procedures must be observed when handling the substance. Only experienced and 					properly trained persons should handle gases. Consider pressure relief equipment in gas 					installations. Ensure that the complete gas system is (or regularly is) checked for leaks 					before use. Do not smoke when using or handling the product. Use only properly specified 					equipment suitable for the product and the prevailing pressure and temperature. Consult 					your supplier if in doubt. Avoid back-flow of water, acids or bases. Do not inhale gas. Avoid 					release of product to the workplace.

Safe use of the gas holder: 			Consult the manual of the holder provided by the supplier. Prevent backflow into the 					holder. Protect the holder from mechanical damage. Do not drag, roll, slide or drop. When 					moving a gas cylinder, even for short distances, always use a (hand)cart that is suitable for 					transporting gas cylinders. Do not remove valve protection caps until the holder is secured 					to a wall, work surface or cylinder holder and is ready for use. If the user experiences any 					difficulty in operating the valve, discontinue use and contact the supplier. Never repair 					holder valves or safety pressure relief equipment yourself. Damaged cylinder valves must 					be reported to the supplier immediately. Keep container valves clean and free from 					contamination, particularly oil and water. Fit plug or bung and container cap, where 					available, as soon as possible after the container has been disconnected from equipment. 					Close the container valve after each use and when empty, even if still connected. Do not 					attempt to transfer gas from one container to another. Never use a flame or electric 					heater to increase the pressure in the container. Do not remove or damage identification 					labels provided by the supplier. Ingress of moisture into the container must be prevented. 					Open the valve slowly to avoid a pressure shock.

7.2. Conditions for safe storage, including any incompatibilities

Conditions for safe storage,			Observe all ordinances and local regulations regarding storage.

including incompatible products	: 		Do not store container in conditions which promote corrosion. Protective plug of tap or 					cap should be fitted. Store container vertically and secure against tipping. Stored 						containers should be checked regularly for leaks and general condition. Store container 					below 50°C in a well ventilated area. Store containers in a location free from fire hazard 					and away from sources of heat and ignition. Keep away from flammable materials.

7.3. Specific end use

No

7.4. Delivery methods

7.4.1 In gaseous form:

7.4.1.1  : In Cylinders under high pressure:

| CONTENTS | DIAMETER (mm) | HEIGHT (mm) | EMPTY WEIGHT | KG CO2 |
| --- | --- | --- | --- | --- |
| 50 L * | 229 | 1700 | ± 67 Kg | 37.5 |
| 27 L * | 204 | 1250 | ± 45 Kg | 20 |
| 13 L | 204 | 700 | ± 23 Kg | 10 |
| 13 L | 140 | 1200 | ± 23 Kg | 10 |
| 7 L | 140 | 700 | ± 12 Kg | 5 |
| 4 L | 140 | 600 | ± 9 Kg | 4 |
| 3 L | 110 | 480 | ± 7 Kg | 2 |
| 3 L | 100 | 570 | ± 6 Kg | 2 |
| 2 L | 100 | 460 | ± 2.5 Kg | 1.5 |
| 1 L | 90 | 410 | ± 1.5 Kg | 1 |
| 0.5 L | 60 | 400 | ± 1 Kg | 0.5 |
| * Also available with riser tube for liquid withdrawal (marked by the red line on the cylindrical part and the letter P on the ogee) |  |  |  |  |

7.4.1.2: In Bottle Bundles:

| VOLUME | SIZE (m) | HEIGHT (m) | EMPTY WEIGHT (Kg) | VOLUME CO2 (Kg) |
| --- | --- | --- | --- | --- |
| 16x 50 L | 1,05 x 1,05 | 2,10 | ± 1500 | 600 |

7.4.2 In liquid, deep-cooled form:

Packaging					Vacuum insulated tanks/containers (documentation on request) 15bar/-25°C

Loading tanks 645l

tankers/containers

High pressure spheres: 2300 kg

Dewars

7.4.3 CO2 as a feed gas:

Conforms to EU231/2012, HACCP system, E-number: E290, Food grade: Purity 4.5

7.4.4 Medical CO2

The product specification complies with the current version 'European Pharmacopoeia': PhEUR 11/2025:2408 monograph, https://www.edqm.eu/en/web/edqm/european-pharmacopoeia-ph.-eur.-11th-edition

7.5 Physicochemical constants:

Symbol					CO2

Average molecular weight (g/mol)			44,01

Gas density (kg/m³ at 1.013 bar and 0°C)		1,977

Relative density of gas (air=1)			1,53

Sublimation point				-78°C ( 197,7 K)

Critical temperature				t=31°C (304,2K), p= 73,8 bar

Vapour pressure (bar at 20°C)			57

Auto-ignition temperature			not flammable

1 L of liquid CO2 releases 500 L of gas at 15°C and 1 bar

7.6 Cylinder identification:

Colour (according to NBN-EN 1089)		the cylindrical part is painted in dark gray RAL 7031

The ogive is painted in light gray RAL 7037

Tap connection standard			W21.8 x 1/14” – right/external (NBN226/DIN477)

Crane material				brass – with residual pressure valve

Identification				ISO sticker with ADR data and H and P phrases according to CLP

Dark gray				Lichtgrijs

7.7 Specification/Quality:

| FORM | NAME | CO2 vol % | H2O vol.ppm | O2 + N2 vol.ppm | O2 vol.ppm | N2 vol.ppm | CNHM vol.ppm | CO vol.ppm | Tot. S vol.ppm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gaseous or with riser (liquid) | 3.0 | ≥ 99,9 | ≤ 10 | ≤ 300 | - | - | ≤ 10 | ≤ 1 | - |
|  | 4,5 * | ≥ 99,995 | ≤ 5 | - | ≤ 5 | ≤ 20 | ≤ 10 | - | - |
|  | Med.CO2 | Min. 99,5 | ≤ 67 | - | - | - | - | ≤ 5 | ≤ 1 |
| Liquid, deep chilled (bulk) ** | 3.0 | ≥ 99,9 | See specification Section 14 |  |  |  |  |  |  |
| Designation according to EN ISO14175: C. |  |  |  |  |  |  |  |  |  |

The specifications are based on the evaporated liquid phase

* Cylinders with sticker designation 4.5 (food grade)

** transport and storage of vacuum insulated containers, thermos

SECTION 8: Exposure Controls/Personal Protection

8.1. Control parameters

8.1.1 National occupational exposure values ​​and biological limit values

| Carbon dioxide (124-38-9) |  |  |
| --- | --- | --- |
| EI-Indicative Occupational Exposure Limit (IOEL) |  |  |
| Local Name | Carbon dioxide |  |
| IEOL TWA | 9000 mg/m³ |  |
| IOEL TWA [ppm] | 5000 ppm |  |
| Reference legislation | COMMISSION DIRECTIVE 2006/15/EC |  |
| Belgium – Occupational exposure limits |  |  |
| Local Name | Kooldioxide |  |
| TGG-8u (OEL TWA) | 9000 mg/m³ |  |
| TGG-8u (OEL TWA) [ppm] | 5000 ppm |  |
| Reference Legislation | Social Legislation |  |

8.1.2. Recommended monitoring procedures

No additional information available

8.1.3. Formed Air Pollutants

No additional information available

8.1.4. DNEL and PNEC

| Carbon dioxide (124-38-9) |  |
| --- | --- |
| DNEL/DMEL (additional information) |  |
| Additional information | None available |
| PNEC (additional information) |  |
| Additional information | None available |

8.1.5. Control banding

No additional information available

8.2. Exposure Control Measures

8.2.1. Appropriate technical measures

Appropriate technical measures:
CO2 detectors should be used where CO2 may be released. Provide adequate general and local exhaust ventilation. Oxygen detectors should be used where quantities of asphyxiating gases may be released. Engineered systems should be regularly tested for leaks. Ensure exposure is below occupational exposure limits (where available). Consider using a system of permits to work, e.g. for maintenance work.

8.2.2. Personal protective equipment

Personal protective equipment:

A safety assessment should be carried out and documented at each work area to assess the risks associated with the use of the product and to select the appropriate PPE for the relevant risks. The following recommendations should be considered: Select PPE that meets the recommended EN/ISO standards.

8.2.2.1. Eye and face protection

Eye protection:

Wear goggles when transferring or breaking transfer connections. Standard EN 166 - eye protection - specifications

8.2.2.2. Skin protection

Hand protection:
Wear working gloves when handling gas cylinders. Standard EN 388- Gloves against mechanical hazards, performance level 1 or higher. Wear cold insulating gloves when transferring or breaking transfer connections. Standard EN 511- Cold insulating gloves.

Other Skin Protection

Wear safety footwear when handling pressure containers. Standard EN ISO 20345 – Personal protective equipment : Safety footwear.

Overige informatie:

Wear safety footwear when handling pressure containers. Standard EN ISO 20345 - Personal protective equipment : Safety footwear.

8.2.2.3. Respiratory protection

Respiratory protection:

Compressed air mask or compressed air line with mask can be used in oxygen-poor atmosphere.

Compressed air mask is indicated where unknown exposure can be expected. For example during maintenance of installation.

Standard EN137 - Self-contained compressed air masks. Contact respiratory protection supplier for selection of suitable material.

8.2.2.4. Thermal hazards

Protection against thermal hazards:

None in addition to the above sections.

8.2.3. Control of environmental exposure

Environmental exposure controls:

Not necessary.

SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

Appearance

Physical condition: 				Gas

Colour: 					Colourless

Form: 					Liquid gas

Smell: 					Odourless

Odor threshold: 				Odour perception threshold is subjective and not suitable as a warning of overexposure

Melting point: 				-78.5 °C Melting point under normal conditions does not exist. At atmospheric pressure, 					solid carbon dioxide sublimates into gaseous carbon dioxide at -78.5°C

Freezing point:				 -56,6 °C

Boiling point:				 -56,6 °C

Flammability: 				Not flammable

Oxidizing properties: 				No oxidizing properties

Explosion limits: 				Unknown

Lowest Explosive Limit Value: 			Not applicable

Upper explosion limit value: 			Not applicable

Flash point: 				Not applicable for gases and gas mixtures

Auto-ignition temperature: 			Not flammable

Decomposition temperature: 			Not applicable

pH: 					Not applicable for gases and gas mixtures

Viscosity, kinematic: 				Not applicable for gases and gas mixtures

Viscosity, dynamic:				0.07 mPa s literature; Not applicable for gases and gas mixtures

Solubility in water: 				2000 mg/l

Partition coefficient n-octanol/water (Log Kow): 	0,83

Partition coefficient n-octanol/water (Log Pow): 	0,83

Dampspanning: 				57,3 bar(a) EC-TEMP: 10

Dampdruk bij 50°C: 				No reliable data available

Critical pressure: 				7375 kPa

Density: 					0,771 g/cm³ 50

Relative density: 				0,82

Relative vapour density at 20°C:	 		Not applicable

Relative gas density: 				1,52

Particle characteristics:				Not applicable

Not applicable for gases and gas mixtures

Nanoforms are not relevant for gases and gas mixtures

9.2. Other information

9.2.1. Information on physical hazard classes

Critical temperature: 				31 °C

9.2.2. Other safety features

Molecular mass: 				44 g/mol

Gas group: 					Press. Gas (Liq.)

Sublimation point: 				-78,5

Additional information: 			Gas/vapour heavier than air. May accumulate in confined spaces, especially in pits, cellars, 					etc.

SECTION 10: Stability and reactivity

10.1. Reactivity

No reactivity hazard other than described in sub-section below

10.2. Chemical stability

Stable under normal conditions

10.3. Possibility of hazardous reactions

No

10.4. Conditions to avoid

Avoid moisture in installation systems

10.5. Chemically Interacting Materials

For further information regarding compatibility, see ISO 11114

10.6. Hazardous decomposition products

No

SECTION 11: Toxicological information

11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008

Acute toxicity: 				Toxicological effects are not expected from this product if the occupational exposure 					limits are not exceeded

Acute toxicity (dermal): 			Not classified

Acute toxiciteit (inhalatie): 			Not classified

Skin corrosion/irritation: 	No known effects of this product
pH: Not applicable for gases and gas mixtures.

Serious eye damage/eye irritation: 	No known effects of this product
pH: Not applicable for gases and gas mixtures

Respiratory/skin sensitization: 			No known effects of this product

Germ cell mutagenicity: 			No known effects of this product

Carcinogenicity:  				No known effects of this product

Reproductive toxicity: 				Not classified

Toxic to reproduction: fertility: 			No known effects of this product

Toxic to reproduction: unborn child: 		No known effects of this product

STOT single exposure: 				No known effects of this product

STOT - repeated exposure: 			No known effects of this product

Danger of inhalation: 				Not applicable for gases and gas mixtures

| Carbon dioxide (124-38-9) |  |
| --- | --- |
| Viscosity, kinematic | Not applicable for gases and gas mixtures. |

11.2. Information about other hazards

11.2.1. Endocrine disrupting properties

Harmful effects of those hormone disrupting properties 	The substance/mixture has no endocrine disrupting properties

on health

11.2.2. Other information

Other information: 				Unlike simple asphyxiating gases, carbon dioxide has the property of causing death even at 					normal oxygen levels (20-21%). At 5% CO2 a synergistic effect has been observed which 					can increase the toxicity of certain gases (CO, NO2). It has been shown that CO2 can 					increase the production of carboxy- and met-hemoglobin by these gases due to the 					stimulatory effects of CO3 on respiration and circulation, For further information see 'EIGA 					Safety Info 24: Carbon Dioxide, Physiological Hazards' at www.eiga.eu. ,The substance / 					mixture has no endocrine disrupting properties.

SECTION 12: Ecological information

12.1. Toxicity

Evaluation: 				This product does not cause any environmental damage

Danger to the aquatic environment, (acute) short term	Not classified

Hazardous to the aquatic environment, (chronic) 	Not classified Not rapidly degradable

long term

| Carbon Dioxide (124-38-9)² |  |
| --- | --- |
| LC50 96 Hours – fish [mg/l] | No data available |
| EC50 48 Hours – Daphnia magna [mg/l] | No data available |
| EC50 72 Hours – Algae [ml/l] | No data available |

12.2. Persistence and degradability

| Carbon dioxide (124-38-9) |  |
| --- | --- |
| Evaluation | This product does not cause any environmental damage |

12.3. Bioaccumulation

| Carbon dioxide (124-38-9) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 0,83 |
| Partition coefficient n-octanol/water (Log Kow) | 0,83 |
| Evaluation | This product does not cause any environmental damage |

12.4. Mobility in the soil

| Carbon dioxide (124-38-9) |  |
| --- | --- |
| Evaluation | This product does not cause any environmental damage |

12.5. Results of PBT and vPvB assessment

Evaluation: 				Not classified as PBT or vPvB

12.6. Endocrine disrupting properties

Other harmful effects: 				No known effects of this product

Evaluation: 				The substance/mixture has no endocrine disrupting properties

12.7. Other harmful effects

Other harmful effects: 				No known effects of this product

Effect on ozone layer: 				No effect on the ozone layer

Global warming factor [CO2=1]: 			1

Effect on global warming: 			Released in large quantities may contribute to the greenhouse effect. Contains one (or 					more) greenhouse gases

SECION 13: Removal Instructions

13.1. Waste treatment methods

Waste treatment methods: 			Discharge to atmosphere in large quantities should be avoided. Discharge only in very well 					ventilated areas. Do not discharge where accumulation could be dangerous. Return 					unused product in original container to supplier

List of hazardous waste. (From Decision 2000/532/EC,	16 05 05: Gases in pressure containers other than those mentioned in 16 05 04

as amended)

13.2. Additional information

External processing and disposal of waste must comply with applicable local and/or national regulations

SECTION 14: Information regarding transportation

Compliant with ADR / IMDG / IATA / ADN / RID

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1 UN Number or ID Number |  |  |  |  |
| UN 1013 | UN 1013 | UN 1013 | UN 1013 | UN 1013 |
| 14.2 Proper shipping name according to the UN model regulations |  |  |  |  |
| CARBON DIOXIDE (CARBON DIOXIDE) (CARBONIC OXYGEN) | CARBON DIOXIDE | Carbon dioxide | CARBON DIOXIDE (CARBON DIOXIDE) (CARBONIC OXYGEN) | CARBON DIOXIDE (CARBON DIOXIDE) (CARBONIC OXYGEN) |
| Description of transport document |  |  |  |  |
| 2.2 | 2.2 | 2.2 | 2.2 | 2.2 |
| 14.4. Packaging group |  |  |  |  |
| See specifications in point 7.4 |  |  |  |  |
| 14.5. Environmental hazards |  |  |  |  |
| Environmentally hazardous: No | Environmentally hazardous: No Marine pollutant: No | Environmentally hazardous: No | Environmentally hazardous: No | Environmentally hazardous: No |
| No additional information available |  |  |  |  |

14.6. Special precautions for user

Specific precautions during transport: 		Avoid transport in vehicles where the load area is not separated from the driver's 					compartment, Ensure that the driver is aware of the potential hazards of the load and 					knows what to do in the event of an accident or emergency, Before transporting cylinders: 					- Ensure adequate ventilation, - Ensure that containers are properly secured, - Check that 					the tap is properly closed and does not leak, - Check that the blind nut or plug (if present) 					is securely fastened, - Check that the tap protection (if present) is securely fastened.

Road transport

Classification code (ADR): 			2A

Special provisions (ADR): 			378, 392, 584, 653, 662

Limited Quantities (ADR): 			120ml

Exempt quantities (ADR): 			E1

Packaging instructions (ADR): 			P200

Mixed Packaging Regulations (ADR): 		MP9

Instructions for portable tanks and bulk containers (ADR):	(M)

Tank code (ADR):				PxBN(M)

Special provisions for tanks (ADR):			TA4, TT9

Vehicle for tank transport:			AT

Transport category (ADR):			3

Special provisions for carriage - Loading, unloading 	CV9, CV10, CV36

and handling (ADR)

Hazard identification number (Kemler no.):		20

20

1013

Orange identification sign

Code for Tunnel Restrictions (ADR):		C/E

Transport on the open sea

Special provision (IMDG):			378

Limited quantities (IMDG):			120 ml

Excepted quantities (IMDG):			E1

Packaging instructions (IMDG):			P200

No. NS (Fire):				F-C

No. NS (Waste):				S-V

Stowage category (IMDG):			A

Measures and observations (IMDG):		Liquefied, non-flammable gas. Heavier than air (1.5). Cannot remain in the liquid state 					above 31°C

Air transport
PCA Expected Quantities (IATA):			E1

PCA Limited Quantities (IATA):			FORBIDDEN

PCA limited quantity max. net quantity (IATA):		FORBIDDEN

PCA packaging regulations (IATA):			200

PCA max. net quantity (IATA): 			75kg

CAO packaging regulations (IATA): 			200

CAO max. net quantity (IATA):			150kg

Special provisions (IATA) : A202 ERG code (IATA):	2L

Transport on inland waters

Classification code (ADN): 			2A

Special provision (ADN): 			378, 392, 584, 653, 662

Limited quantities (ADN): 			120 ml

Excepted quantities (ADN): 			E1

Required equipment (ADN): 			PP

Number of blue cones/lights (ADN): 		0

Railway transport

Classification code (RID): 			2A

Special provision (RID):				378, 392, 584, 653, 662

Limited Quantities (RID): 			120ml

Excepted Quantities (RID): 			E1

Packaging Instructions (RID): 			P200

Special provisions for mixed packaging (RID):		MP9

Instructions for portable tanks and bulk containers (RID): 	(M)

Tank codes for RID tanks (RID):			PxBN(M)

Special provisions for RID tanks (RID): 		TA4, TT9, TM6

Transport category (RID):			3

Special provisions for carriage - Loading, unloading	CW9, CW10, CW36

and handling (RID)

Express Parcel (RID):				CE3

Hazard Identification Number (RID): 		20

14.7. Bulk maritime transport in accordance with IMO instruments

IBC-code: 					Not applicable

SECTION 15: Regulations

15.1. Specific safety, health and environmental regulations and legislation for the substance or mixture15.1.1. EU regulations

REACH Annex XVII (substances with restricted use)

Not included in REACH Annex XVII

REACH Annex XIV (list of substances subject to authorisation)

Not included in REACH Annex XIV (list of substances subject to authorisation)

REACH Candidate List (SVHC)

Niet opgenomen in de REACH kandidatenlijst

PIC Regulation (Prior Informed Consent)

Not included in the PIC list (Regulation EU 649/2012)

POP Regulation (Persistent Organic Pollutants)

Not included in the POP (Regulation EU 2019/1021)

Ozone Regulation (1005/2009)

Not included in the list of ozone depleting substances (Regulation EU 1005/2009)

VOC Directive (2004/42)

Usage Restrictions: 				No.

Seveso Regulation (Disaster Risk Reduction)

Seveso Directive 2012/18/EU (Seveso III):		not cited

Explosives Precursors Regulation (2019/1148)

Does not contain substances included in the list of explosives precursors (Regulation EU 2019/1148 on the marketing and use of explosives precursors)

Regulation on medicinal precursors (273/2004)

Does not contain substances included in the list of precursors for medicines (Regulation EC 273/2004 on the production and placing on the market of certain substances used in the illicit manufacture of narcotic drugs and psychotropic substances)

15.1.2. National regulations

Ensure that all national/local regulations are known.

Safety data sheet in accordance with Regulation (EU) No 2020/878.

Council Directive 89/391/EEC on the implementation of measures to encourage improvements in the safety and health of workers at work.

Directive 2016/425/EEC on personal protective equipment.

Directive 2014/34/EU on equipment and protective systems intended for use in potentially explosive atmospheres (ATEX).

Only products that comply with the Food Regulations (EC) No. 1333/2008 and (EU) No. 231/2012 and are labelled as such may be used as food additives.

This safety data sheet is prepared in accordance with Regulation (EU) No. 2015/830.

Belgium

ABM category: 				B(4) - Slightly harmful to aquatic organisms

SZW list of carcinogenic substances: 		The substance is not present

SZW-lijst van mutagene stoffen: 			The substance is not present

SZW-lijst van reprotoxische stoffen – Borstvoeding: 	The substance is not present

SZW-lijst van reprotoxische stoffen – Vruchtbaarheid: 	The substance is not present

SZW-lijst van reprotoxische stoffen – Ontwikkeling: 	The substance is not present

15.2. Chemical Safety Assessment

A Chemical Safety Assessment (CSA) does not need to be carried out for this product

SECTION 16: Other information

Indication of changes: Safety data sheet in accordance with Regulation (EU) No 2020/878

| Indication of changes |  |
| --- | --- |
| Modified item | Change Comments |

| Abbreviations and acronyms: |  |
| --- | --- |
| ADN | European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways |
|  | ADR - Convention concerning the International Carriage of Dangerous Goods by Road. |
|  | ATE - Acute Toxicity Estimate - acute toxicity estimates |
| BLV | Biological limit value |
| BOD | Biochemical oxygen demand (BOD) |
| CAO | Cargo Aircraft only / Cargo aircraft only. |
| CAS-Nr | Chemical Abstract Service - Number |
| CLP | Classification Labelling Packaging; Regulation (EC) No 1272/2008 on classification, labelling and packaging. |
| COD | Chemical oxygen demand (COD) |
| CSA | CSA - Chemical Safety Assessment. |
| DMEL | Derived Minimal Effect Dose |
| DNEL | Derived No-Effect Level |
| EC50 | Median effective concentration |
| EC | European Inventory of Existing Commercial Chemical Substances |
| ED | Endocrine disrupting properties |
| EINECS | EINECS - European Inventory of Existing Commercial Chemical Substances -European list of existing commercial substances |
| EN | European standard |
| IARC | International Centre for Cancer Research |
| IATA | International Air Transport Association |
| IMDG | International Maritime Dangerous Goods Code |
| IOELV | Indicative Occupational Exposure Limit |
| LC50 | Concentration that causes death in 50% of a test population |
| LD50 | Dose that causes death in 50% of a test population (median lethal dose) |
| LOAEL | Lowest dose or concentration at which an adverse effect was observed |
| NOAEC | Concentration at which no adverse effect was observed |
| NOAEL | Dose or concentration at which no adverse effect was observed |
| NOEC | Concentration with no observed effects |
| N.E.G. | Not Elsewhere Mentioned |
| OECD | Organisation for Economic Co-operation and Development, OECD |
| OEL | Occupational exposure limit |
| PBT | Persistent, bioaccumulative and toxic substance |
| PCA | Passenger and Cargo Aircraft / Passenger and cargo aircraft |
| PNEC | Predicted No Effect Concentration(s) |
| PBM | PPE – Personal Protective Equipment. |
| REACH | Regulation (EC) No 1907/2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals |
| RID | Regulations concerning the international carriage of dangerous goods by rail |
| RMM | Risk Management Measures - Risk Management Measures |
| STP | Water treatment plant |
| ThZV | Theoretical oxygen demand (THD) |
| TLM | Median Tolerance Limit |
| TRGS | Technical measures for hazardous substances |
| STOT-RE | Specific Target Organ Toxicity-Repeated Exposure |
| STOT-SE | Specific Target Organ Toxicity-Single Exposure |
| UFI | Unique Formula Identifier / Unique Formula Identifier |
| UN | UN - VN - United Nations |
| VOS | Volatile organic compounds |
| zPzB | Very persistent and very bioaccumulative, vPvB |
| WGK | Water hazard class |
| Full text of sentences H and EUH: |  |
| H280 | Contains gas under pressure; may explode if heated. |
| Press. Gas (Liq.) | Gases under pressure : Liquid gas |

The layout is in accordance with:			ATP 12

DISCLAIMER OF LIABILITY: 			Before this substance may be used in a new process or experiment, a careful material 					compatibility and safety study should be performed. The data given here in this document 					are believed to be correct at the time of publication. However, the publisher accepts no 					liability for damage of any kind arising from the use of data from this sheet.