Issue date: 1-7-2017   Revision date: 1-10-2025   Supersedes version of: 3-7-2023   Version: 6.1

Danger

Danger

# SECTION 1: Identification of the substance/mixture and of the company/undertaking

## 1.1. Product identifier

| Trade name | : Acetylene (dissolved) - (cylinders) |
| --- | --- |
| SDS no | : EIGA001-ACE-ALBNL |
| Other means of identification | : Acetylene (dissolved) - (cylinders) |
|  | CAS-No. : 74-86-2 |
|  | EC-No. : 200-816-9 |
|  | EC Index-No. : 601-015-00-0 |
| REACH registration No | : 01-2119457406-36 |
| Chemical formula | : C2H2 |

## 1.2. Relevant identified uses of the substance or mixture and uses advised against

Relevant identified uses 	: See the list of identified uses and exposure scenarios in the annex of the safety data sheet.

Consumer use.

Perform risk assessment prior to use.

Uses advised against 	: None.

## 1.3. Details of the supplier of the safety data sheet

BELGIUM:

Janox Bv

Roggestraat 12

8972 Krombeke

Belgium-Belgique-België

Tel: +32 (0)57 30 98 40

info@janox.be

## 1.4. Emergency telephone number

| Country/Area | Organisation/Company | Address | Emergency number | Comment |
| --- | --- | --- | --- | --- |
| Belgium | Centre Anti-Poisons/Antigifcentrum c/o Hôpital Militaire Reine Astrid | Rue Bruyn 1 1120 Bruxelles/Brussel | +32 70 245 245 | Please dial: 070 245 245 for any urgent questions about intoxication (free of charge 24/7), if not accessible, dial: 02 264 96 30 (standard fee) |
| Luxembourg | Centre Anti-Poisons/Antigifcentrum c/o Hôpital Militaire Reine Astrid | Rue Bruyn 1 1120 Bruxelles/Brussel | +352 8002 5500 | Free telephone number with a 24/7 access. Experts answer all urgency questions on dangerous products in French, Dutch and English |
| Netherlands | Nationaal Vergiftigingen Informatie Centrum | Huispostnummer Q03.2.315 Postbus 85500 3508 GA Utrecht | +31 88 755 80 00 | Only for the purpose of informing medical personnel in cases of acute intoxications (24 hours a day, 7 days a week) |

# SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

## Classification according to Regulation (EC) No. 1272/2008 [CLP]

Physical hazards 	Flammable gases, Category 1A, Chemically unstable gas H220;H230

A Gases under pressure : Dissolved gas 	H280

2.2. Label elements

## Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Hazard pictograms (CLP) 	:

|  | GHS02 GHS04 |
| --- | --- |
| Signal word (CLP) | : Danger |
| Hazard statements (CLP) | : H220 - Extremely flammable gas. H280 - Contains gas under pressure; may explode if heated. H230 - May react explosively even in the absence of air. |
| Precautionary statements (CLP) |  |
| - Prevention | : P202 - Do not handle until all safety precautions have been read and understood. P210 - Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| - Response | : P377 - Leaking gas fire: Do not extinguish, unless leak can be stopped safely. P381 - In case of leakage, eliminate all ignition sources. |

- Storage 	: P403 - Store in a well-ventilated place.

Supplemental information 	: Dispose of cylinder via gas supplier only. Cylinder contains a porous material which in some cases contains asbestos fibres.

### 2.3. Other hazards

Asphyxiant in high concentrations.

These high concentrations  are within the flammability range.

Not classified as PBT or vPvB.

The substance/mixture has no endocrine disrupting properties.

Not classified as PMT or vPvM.

# SECTION 3: Composition/information on ingredients

## 3.1. Substances

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] ATE, EUH-statements, M-Factors |
| --- | --- | --- | --- |
| Acetylene (dissolved) - (cylinders) | CAS-No.: 74-86-2 EC-No.: 200-816-9 EC Index-No.: 601-015-00-0 REACH registration No: 01-2119457406- 36 | 100 | Flam. Gas 1A - Chem. Unst. Gas A, H220;H230 Press. Gas (Diss.), H280 |

The cylinder contains a porous material which in some cases contains asbestos fibres. Asbestos  is subject to restrictions on its use (Annex XVII of REACH). The asbestos fibres are encapsulated in the solid porous material and are not released under normal conditions of use. See section 13 for the disposal of those cylinders.

For safety reasons, the acetylene is dissolved in acetone (Flam. Liq. 2, Eye Irrit. 2, STOT SE 3) in the gas receptacle. Vapour of the solvent is carried away as impurity when the acetylene is extracted from the gas receptacle. The concentration of the solvent vapour in the gas is lower than the concentration limits to change the classification of the acetylene.

Full text of H- and EUH-statements: see section 16

Contains no other components or impurities which will influence the classification of the product.

3.2. Mixtures 	 Not applicable

# SECTION 4: First aid measures

## 4.1. Description of first aid measures

- Inhalation 	: Remove victim to uncontaminated area wearing self contained breathing apparatus. Keep

victim warm and rested. Call a doctor. Perform cardiopulmonary resuscitation if breathing stopped.

- Skin contact 	: Adverse effects not expected from this product.

- Eye contact 	: Adverse effects not expected from this product.

- Ingestion 	: Ingestion is not considered a potential route of exposure.

## 4.2. Most important symptoms and effects, both acute and delayed

In high concentrations may cause asphyxiation. Symptoms may include loss of

mobility/consciousness. Victim may not be aware of asphyxiation. See section 11.

4.3. Indication of any immediate medical attention and special treatment needed

None.

# SECTION 5: Firefighting measures

## 5.1. Extinguishing media

| - Suitable extinguishing media | : Water spray or fog. Dry powder. Carbon dioxide. Shutting off the source of the gas is the preferred method of control. Be aware of the risk of formation of static electricity with the use of CO2 extinguishers. Do not use them in places where a flammable atmosphere may be present. |
| --- | --- |
| - Unsuitable extinguishing media | : Do not use water jet to extinguish. |

## 5.2. Special hazards arising from the substance or mixture

| Specific hazards | : Exposure to fire may cause containers to rupture/explode. |
| --- | --- |
| Hazardous combustion products 5.3. Advice for firefighters | : Carbon monoxide. |
| Specific methods | : Use fire control measures appropriate for the surrounding fire. Exposure to fire and heat radiation may cause gas receptacles to rupture. Cool endangered receptacles with water spray jet from a protected position. Prevent water used in emergency cases from entering sewers and drainage systems. If possible, stop flow of product. Use water spray or fog to knock down fire fumes if possible. Do not extinguish a leaking gas flame unless absolutely necessary. Spontaneous/explosive re-ignition may occur. Extinguish any other fire. Continue water spray from protected position until container stays cool. Move containers away from the fire area if this can be done without risk. |
| Special protective equipment for fire fighters | : In confined space use self-contained breathing apparatus. Standard protective clothing and equipment (Self Contained Breathing Apparatus) for fire fighters. Standard EN 137 - Self-contained open-circuit compressed air breathing apparatus with full face mask. Standard EN 469 - Protective clothing for firefighters. Standard - EN 659: Protective gloves for firefighters. EN 15090 Footwear for firefighters. EN 443 Helmets for fire fighting in buildings and other structures. |

# SECTION 6: Accidental release measures

## 6.1. Personal precautions, protective equipment and emergency procedures

| For non-emergency personnel | : Act in accordance with local emergency plan. Try to stop release. Evacuate area. Eliminate ignition sources. Ensure adequate air ventilation. Stay upwind. See section 8 of the SDS for more information on personal protective equipment. |
| --- | --- |
| For emergency responders 6.2. Environmental precautions | : Monitor concentration of released product. Consider the risk of potentially explosive atmospheres. Wear self-contained breathing apparatus when entering area unless atmosphere is proved to be safe. See section 5.3 of the SDS for more information. |
|  | Try to stop release. |

6.3. Methods and material for containment and cleaning up

Ventilate area.

6.4. Reference to other sections

See also sections 8 and 13.

# SECTION 7: Handling and storage

## 7.1. Precautions for safe handling

| Safe use of the product | : Do not breathe gas. Avoid release of product into atmosphere. The product must be handled in accordance with good industrial hygiene and safety procedures. Only experienced and properly instructed persons should handle gases under pressure. Consider pressure relief device(s) in gas installations. Ensure the complete gas system was (or is regularily) checked for leaks before use. Do not smoke while handling product. Use only properly specified equipment which is suitable for this product, its supply pressure and temperature. Contact your gas supplier if in doubt. Avoid suck back of water, acid and alkalis. Assess the risk of potentially explosive atmospheres and the need for explosion-proof equipment. Purge air from system before introducing gas. Take precautionary measures against static discharge. Keep away from ignition sources (including static discharges). Consider the use of only non-sparking tools. Avoid contact with pure copper, mercury, silver and brass with greater than 65% copper. Operating pressure in piping should be limited to 1.5 bar (gauge) or less due to more stringent national regulations (with maximum diameter DN25). Consider the use of flash back arrestors. Solvent may accumulate in piping systems. For maintenance activities use appropriate resistant gloves, assess the necessity to use a respiratory filter device (specify gloves and filters for DMF or acetone use) and wear safety goggles. Avoid breathing the vapour of the solvent. Provide adequate ventilation. For further information on safe use refer to EIGA code of practice acetylene (EIGA Doc 123). Ensure equipment is adequately earthed. |
| --- | --- |
| Safe handling of the gas receptacle | : Suck back of water into the container must be prevented. Open valve slowly to avoid pressure shock. Refer to supplier's container handling instructions. Do not allow backfeed into the container. Protect containers from physical damage; do not drag, roll, slide or drop. Do not remove or deface labels provided by the supplier for the identification of the content of the container. When moving cylinders, even for short distances, use a cart (trolley, hand truck, etc.) designed to transport cylinders. Leave valve protection caps, when provided, in place until the container has been secured against either a wall or bench or placed in a container stand and is ready for use. If user experiences any difficulty operating valve discontinue use and contact supplier. Close container valve after each use and when empty, even if still connected to equipment. Never attempt to repair or modify container valves or safety relief devices. Damaged valves should be reported immediately to the supplier. Replace valve outlet caps or plugs and container caps where supplied as soon as container is disconnected from equipment. Keep container valve outlets clean and free from contaminants particularly oil and water. Never attempt to transfer gases from one cylinder/container to another. Never use direct flame or electrical heating devices to raise the pressure of a container. |

## 7.2. Conditions for safe storage, including any incompatibilities

Observe all regulations and local requirements regarding storage of containers.

Containers should not be stored in conditions likely to encourage corrosion.

Container valve guards or caps, when provided, should be in place.

Containers should be stored in the vertical position and properly secured to prevent them from falling over.

Stored containers should be periodically checked for general condition and leakage.

Keep container below 50°C in a well ventilated place.

Store containers in location free from fire risk and away from sources of heat and ignition.

Keep away from combustible materials.

Segregate from oxidant gases and other oxidants in store.

All electrical equipment in the storage areas should be compatible with the risk of a potentially explosive atmosphere.

7.3. Specific end use(s)

None.

# SECTION 8: Exposure controls/personal protection

## 8.1. Control parameters

| Acetylene (dissolved) - (cylinders) (74-86-2) |  |
| --- | --- |
| Belgium - Occupational Exposure Limits |  |
| Local name | Acétylène # Acetyleen |
| Remark | A: la mention “A” signifie que l’agent libère un gaz ou une vapeur qui n’ont en eux-mêmes aucun effet physiologique mais peuvent diminuer le taux d’oxygène dans l’air. Lorsque le taux d’oxygène descend en dessous de 1718 % (vol/vol) le manque d’oxygène provoque des suffocations qu’aucun symptôme préalable n’annonce. # A: de vermelding “A” betekent dat dit agens gas of damp vrijgeeft dat of die op zich geen fysiologische werking heeft, maar het zuurstofgehalte in de lucht verlaagt. Wanneer het zuurstofgehalte daalt onder de 17-18 % (vol/vol), veroorzaakt het zuurstoftekort verstikking, die zich manifesteert zonder dat er een waarschuwing aan voorafgaat. |
| Regulatory reference | Koninklijk besluit/Arrêté royal 16/11/2023 |

| DNEL (Derived-No Effect Level) | : None established. |
| --- | --- |
| PNEC (Predicted No-Effect Concentration) 8.2. Exposure controls 8.2.1. Appropriate engineering controls | : None established. |
|  | Provide adequate general and local exhaust ventilation. Product to be handled in a closed system. Systems under pressure should be regularily checked for leakages. Ensure exposure is below occupational exposure limits (where available). Gas detectors should be used when toxic gases may be released. Consider the use of a work permit system e.g. for maintenance activities. |

### 8.2.2. Individual protection measures, e.g. personal protective equipment

A risk assessment should be conducted and documented in each work area to assess the

risks related to the use of the product and to select the PPE that matches the relevant risk.

The following recommendations should be considered:

PPE compliant to the recommended EN/ISO standards should be selected.

- Eye/face protection 	: Wear safety glasses with side shields.

Standard EN 166 - Personal eye-protection - specifications.

- Skin protection

  - Hand protection 	: Wear working gloves when handling gas containers.

Standard EN 388 - Protective gloves against mechanical risks, performance level 1 or higher. Recommended types include wrist gloves from leather or synthetic material with equivalent performance, fabric gloves, fabric gloves with leather palms.

  - Other 	: Consider the use of flame resistant anti-static safety clothing.

Standard EN ISO 14116 - Limited flame spread materials.

Standard EN 1149-5 - Protective clothing: Electrostatic properties.

Wear safety shoes while handling containers.

Standard EN ISO 20345 - Personal protective equipment - Safety footwear.

- Respiratory protection 	: Standard EN 137 - Self-contained open-circuit compressed air breathing apparatus with full face mask.

Consult respiratory device supplier´s product information for the selection of the appropriate device.

Self contained breathing apparatus (SCBA) or positive pressure airline with mask are to be used in oxygen-deficient atmospheres.

Self contained breathing apparatus is recommended, where unknown exposure may be expected, e.g. during maintenance activities on installation systems.

- Thermal hazards 	: Wear goggles with suitable filter lenses when use is cutting/welding.

### 8.2.3. Environmental exposure controls

Refer to local regulations for restriction of emissions to the atmosphere. See section 13 for

specific methods for waste gas treatment.

# SECTION 9: Physical and chemical properties

## 9.1. Information on basic physical and chemical properties

Appearance

- Physical state at 20°C / 101.3kPa 	: Gas.

- Colour 	: Colourless.

Odour 	: Garlic like. Poor warning properties at low concentrations.

Melting point / Freezing point 	: -80,8 °C

Boiling point 	: -84 °C

Flammability 	: Extremely flammable gas.

Lower explosion limit 	: 2,3 vol %

Upper explosion limit 	: 100 vol %

Flash point 	: Not applicable for gases and gas mixtures.

Auto-ignition temperature 	: 305 °C

Decomposition temperature : Not applicable. pH 	: Not applicable for gases and gas mixtures.

Viscosity, kinematic 	: No reliable data available.

Water solubility [20°C] 	: 1185 mg/l

Partition coefficient n-octanol/water (Log Kow) 	: 0,37

Vapour pressure [20°C] 	: 44 bar(a)

Vapour pressure [50°C] 	: Not applicable.

Density and/or relative density 	: Not applicable for gases and gas mixtures.

Relative vapour density (air=1) 	: 0,9

Particle characteristics 	: Not applicable for gases and gas mixtures.

Nanoforms are not relevant for gases and gas mixtures.

## 9.2. Other information

### 9.2.1. Information with regard to physical hazard classes

| Tci | : 3 % |
| --- | --- |
| Oxidising properties | : No oxidising properties. |
| - Coefficient of oxygen equivalency (Ci) | : Not applicable. |
| Critical temperature [°C] | : 35 °C |

### 9.2.2. Other safety characteristics

| Molar mass | : 26 g/mol |
| --- | --- |
| Other data | : None. |

# SECTION 10: Stability and reactivity

10.1. Reactivity

No reactivity hazard other than the effects described in sub-sections below.

## 10.2. Chemical stability

Dissolved in a solvent supported in a porous mass.

Stable under recommended handling and storage conditions (see section 7). May react explosively even in the absence of air.

## 10.3. Possibility of hazardous reactions

May decompose violently at high temperature and/or pressure or in the presence of a

catalyst.

Can form explosive mixture with air.

May react violently with oxidants.

May react explosively even in the absence of air.

## 10.4. Conditions to avoid

Keep away from heat/sparks/open flames/hot surfaces. – No smoking.

Avoid moisture in installation systems.

High temperature. High pressure.

## 10.5. Incompatible materials

Forms explosive acetylides with copper, silver and mercury.

Do not use alloys containing more than 65% copper.

Air, Oxidisers.

Do not use alloys containing more than 43% silver.

For additional information on compatibility refer to ISO 11114.

## 10.6. Hazardous decomposition products

Under normal conditions of storage and use, hazardous decomposition products should not

be produced.

# SECTION 11: Toxicological information

## 11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008

| Acute toxicity |  | : There are no data on oral and dermal toxicity (studies are not technically feasible as the substance is a gas at room temperature. |  |  |
| --- | --- | --- | --- | --- |
| Skin corrosion/irritation |  | : No known effects from this product. |  |  |
| Serious eye damage/irritation |  | : No known effects from this product. |  |  |
| Respiratory or skin sensitisation |  | : No known effects from this product. |  |  |
| Germ cell mutagenicity |  | : No known effects from this product. |  |  |
| Carcinogenicity |  | : No known effects from this product. |  |  |
| Toxic for reproduction : Fertility |  | : No known effects from this product. |  |  |
| Toxic for reproduction : unborn child |  | : No known effects from this product. |  |  |
| STOT-single exposure |  | : No known effects from this product. |  |  |
| STOT-repeated exposure |  | : No known effects from this product. |  |  |
| Aspiration hazard 11.2. Information on other hazards |  | : Not applicable for gases and gas mixtures. |  |  |
| Other information |  | : The substance/mixture has no endocrine disrupting properties. |  |  |
| SECTION 12: Ecological information |  |  |  |  |
| 12.1. Toxicity Assessment |  | : | Classification criteria are not met. |  |
| EC50 48h - Daphnia magna [mg/l] |  | : | 242 mg/l |  |
| EC50 72h - Algae [mg/l] |  | : | 57 mg/l |  |
| LC50 96 h - Fish [mg/l] |  | : | 545 mg/l |  |
| 12.2. Persistence and degradability Assessment |  | : | Will rapidly degrade by indirect photolysis in air. Will not undergo hydrolysis. |  |
| 12.3. Bioaccumulative potential Assessment |  | : | Not expected to bioaccumulate due to the low log Kow (log Kow < 4). See section 9. |  |
| 12.4. Mobility in soil Assessment |  | : | Because of its high volatility, the product is unlikely to cause ground or water pollution. Partition into soil is unlikely. |  |
| 12.5. Results of PBT and vPvB assessment Assessment |  | : | Not classified as PBT or vPvB. |  |
| 12.6. Endocrine disrupting properties Assessment |  | : | The substance/mixture has no endocrine disrupting properties. |  |
| 12.7. Other adverse effects Other adverse effects |  | : | Not classified as PMT or vPvM. |  |
| Effect on the ozone layer |  | : | No effect on the ozone layer. |  |
| Effect on global warming |  | : | No known effects from this product. |  |
| SECTION 13: Disposal considerations |  |  |  |  |
| 13.1. Waste treatment methods |  |  | Contact supplier if guidance is required. Do not discharge into areas where there is a risk of forming an explosive mixture with air. Waste gas should be flared through a suitable burner with flash back arrestor. Do not discharge into any place where its accumulation could be dangerous. Ensure that the emission levels from local regulations or operating permits are not exceeded. Refer to the EIGA code of practice Doc.30 "Disposal of Gases", downloadable at http://www.eiga.org for more guidance on suitable disposal methods. Return unused product in original container to supplier. |  |
| List of hazardous waste codes (from Commission Decision 2000/532/EC as amended) |  | : | 16 05 04 *: Gases in pressure containers (including halons) containing hazardous substances. |  |
| 13.2. Additional information |  |  | Dispose of cylinder via gas supplier only. Cylinder contains a porous material which in some cases contains asbestos fibres and is saturated with a solvent (acetone or dimethylformamide). External treatment and disposal of waste should comply with applicable local and/or national regulations. |  |
| SECTION 14: Transport information |  |  |  |  |

## 14.1. UN number or ID number

In accordance with ADR / RID / IMDG / IATA / ADN

UN-No. 	: 1001

## 14.2. UN proper shipping name

| Transport by road/rail/inland waterways (ADR/RID/ADN) | : ACETYLENE, DISSOLVED |
| --- | --- |
| Transport by air (ICAO-TI / IATA-DGR) | : Acetylene, dissolved |
| Transport by sea (IMDG) 14.3. Transport hazard class(es) | : ACETYLENE, DISSOLVED |

Labelling 	:

|  | 2.1 : Flammable gases. |
| --- | --- |
| Transport by road/rail/inland waterways (ADR/RID/ADN) |  |
| Class | : 2 |
| Classification code | : 4F |
| Hazard identification number | : 239 |
| Tunnel Restriction | : B/D - Tank carriage: Passage forbidden through tunnels of category B, C, D and E. Other carriage: Passage forbidden through tunnels of category D and E |
| Transport by air (ICAO-TI / IATA-DGR) |  |
| Class / Div. (Sub. risk(s)) | : 2.1 |
| Transport by sea (IMDG) |  |
| Class / Div. (Sub. risk(s)) | : 2.1 |
| Emergency Schedule (EmS) - Fire | : F-D |
| Emergency Schedule (EmS) - Spillage 14.4. Packing group | : S-U |
| Transport by road/rail/inland waterways (ADR/RID/ADN) | : Not applicable. |
| Transport by air (ICAO-TI / IATA-DGR) | : Not applicable. |
| Transport by sea (IMDG) 14.5. Environmental hazards | : Not applicable. |
| Transport by road/rail/inland waterways (ADR/RID/ADN) | : None. |
| Transport by air (ICAO-TI / IATA-DGR) | : None. |
| Transport by sea (IMDG) 14.6. Special precautions for user | : None. |
| Packing Instruction(s) |  |
| Transport by road/rail/inland waterways (ADR/RID/ADN) | : P200. |
| Transport by air (ICAO-TI / IATA-DGR) |  |
| Passenger and Cargo Aircraft | : Forbidden. |
| Cargo Aircraft only | : 200. |
| Transport by sea (IMDG) | : P200. |
| Special transport precautions | : Avoid transport on vehicles where the load space is not separated from the driver's compartment. Ensure vehicle driver is aware of the potential hazards of the load and knows what to do in the event of an accident or an emergency. Before transporting product containers: - Ensure there is adequate ventilation. - Ensure that containers are firmly secured. - Ensure valve is closed and not leaking. - Ensure valve outlet cap nut or plug (where provided) is correctly fitted. - Ensure valve protection device (where provided) is correctly fitted. |

14.7. Maritime transport in bulk according to IMO instruments

Not applicable.

# SECTION 15: Regulatory information

## 15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture EU-Regulations

| Restrictions on use |  | : None. |  |
| --- | --- | --- | --- |
| Other information, restriction and prohibition regulations |  | : Not listed on the PIC list (Regulation EU 649/2012). Not listed on the POP list (Regulation EU 2019/1021). |  |
| Seveso Directive : 2012/18/EU (Seveso III) |  | : Listed. |  |
| Seveso III Part II (Named dangerous substances) |  | Qualifying quantity (tonnes) |  |  |
|  |  | Lower-tier |  | Upper-tier |
| 19. Acetylene |  | 5 |  | 50 |

## National regulations

| Regulatory reference | : | Ensure all national/local regulations are observed. |
| --- | --- | --- |
| 15.2. Chemical safety assessment |  | A CSA has been carried out. |
| SECTION 16: Other information |  |  |

Indication of changes 	: Revised safety data sheet in accordance with commission regulation (EU) No 453/2010.

| Section | Changed item | Comments |
| --- | --- | --- |
|  | Reference number | Modified |
| 2.3 | Other hazards which do not result in classification | Modified |
| 8 | Regulatory reference | Modified |
| 8.2 | Respiratory protection | Modified |
| 9 | Partition coefficient n-octanol/water (Log Kow) | Added |
| 9 | Density | Added |
| 9 | Tci | Modified |
| 9 | Partition coefficient n-octanol/water (Log Pow) | Added |
| 11.1 | Other information | Added |
| 12.7 | Other adverse effects | Modified |
| 16 | Abbreviations and acronyms | Modified |

| Abbreviations and acronyms | : ATE - Acute Toxicity Estimate. CLP - Classification Labelling Packaging Regulation; Regulation (EC) No 1272/2008. REACH - Registration, Evaluation, Authorisation and Restriction of Chemicals Regulation (EC) No 1907/2006. EINECS - European Inventory of Existing Commercial Chemical Substances. CAS# - Chemical Abstract Service number. PPE - Personal Protection Equipment. LC50 - Lethal Concentration to 50 % of a test population. RMM - Risk Management Measures. PBT - Persistent, Bioaccumulative and Toxic. vPvB - Very Persistent and Very Bioaccumulative. STOT- SE : Specific Target Organ Toxicity - Single Exposure. CSA - Chemical Safety Assessment. EN - European Standard. UN - United Nations. ADR - Agreement concerning the International Carriage of Dangerous Goods by Road. IATA - International Air Transport Association. IMDG code - International Maritime Dangerous Goods. RID - Regulations concerning the International Carriage of Dangerous Goods by Rail. WGK - Water Hazard Class. STOT - RE : Specific Target Organ Toxicity - Repeated Exposure. UFI : Unique Formula Identifier. ADN -International Carriage of Dangerous Goods by Inland Waterways. PROC -Process category . ERC – Environmental release category. PMT - Persistent, Mobile and Toxic. vPvM – very Persistent and very Mobile. |
| --- | --- |
| Training advice | : Ensure operators understand the flammability hazard. |
| Further information | : Classification in accordance with the procedures and calculation methods of Regulation (EC) 1272/2008 (CLP). Key literature references and sources of data are maintained in EIGA doc 169 : 'Classification and Labelling Guide', downloadable at http://www.Eiga.eu . |

| Full text of H- and EUH-statements |  |
| --- | --- |
| Flam. Gas 1A - Chem. Unst. Gas A | Flammable gases, Category 1A, Chemically unstable gas A |
| Press. Gas (Diss.) | Gases under pressure : Dissolved gas |
| H220 | Extremely flammable gas. |
| H230 | May react explosively even in the absence of air. |
| H280 | Contains gas under pressure; may explode if heated. |

DISCLAIMER OF LIABILITY 	: Before using this product in any new process or experiment, a thorough material

compatibility and safety study should be carried out.

Details given in this document are believed to be correct at the time of going to press. Whilst proper care has been taken in the preparation of this document, no liability for injury or damage resulting from its use can be accepted.

# Annex to the safety data sheet

This Annex documents the Exposure Scenarios (ESs) related to the identified uses of the registered substance. The ESs detail protective measures for workers and the environment in addition to those described in sections 7, 8, 11, 12 and 13 of the SDS that are required to ensure that the potential exposure to workers and the environment remains within acceptable levels for each of the identified uses. Table of contents of the Annex

| Identified Uses | Es N° | Short title | ERC | PROC | Page |
| --- | --- | --- | --- | --- | --- |
| Formulation of mixtures in pressure receptacles | EIGA001-1 | Industrial uses, closed contained conditions | ERC2 | PROC1 | 14 |
| Transfilling in pressure receptacles | EIGA001-1 | Industrial uses, closed contained conditions | ERC2 | PROC8b PROC9 | 14 |
| Calibration of analysis equipment | EIGA001-1 | Industrial uses, closed contained conditions | ERC7 | PROC1 PROC2 PROC15 | 14 |
| Feedstock in chemical processes | EIGA001-1 | Industrial uses, closed contained conditions | ERC4 ERC6a ERC6b | PROC1 PROC2 PROC3 | 14 |
| Fuel gas for welding, cutting, heating, brazing and soldering applications. | EIGA001-1 | Industrial uses, closed contained conditions | ERC7 | PROC16 | 14 |
| Fuel gas for welding, cutting, heating, brazing and soldering applications. | EIGA001-2 | Professional uses | ERC9a ERC9b | PROC16 | 17 |
| Fuel gas for welding, cutting, heating, brazing and soldering applications. | EIGA001-3 | Consumer use. | ERC9a ERC9b | PROC13 | 19 |

## 1. EIGA001-1 - Industrial uses, closed contained conditions

### 1.1. Title section

| Industrial uses, closed contained conditions |  |
| --- | --- |
| ES Ref.: EIGA001-1 ES Type: Worker - EIGA Revision date: 2-12-2019 |  |

| Processes, tasks, activities covered | Industrial uses, including product transfers and associated laboratory activities within different closed or contained systems |
| --- | --- |
| Assessment method | ECETOC TRA 2.0 |

### 1.2. Conditions of use affecting exposure

1.2.1. Control of environmental exposure: Manufacture of the substance, Formulation into mixture, Use of non-reactive processing aid at industrial site (no inclusion into or onto article), Use of intermediate, Use of reactive processing aid at industrial site (no inclusion into or onto article), Use of functional fluid at industrial site, Use of functional fluid at industrial site, Widespread use of functional fluid (indoor), Widespread use of functional fluid (outdoor) (ERC1, ERC2, ERC4, ERC6a, ERC6b, ERC7, ERC8d, ERC9a, ERC9b)

| ERC1 | Manufacture of the substance |
| --- | --- |
| ERC2 | Formulation into mixture |
| ERC4 | Use of non-reactive processing aid at industrial site (no inclusion into or onto article) |
| ERC6a | Use of intermediate |
| ERC6b | Use of reactive processing aid at industrial site (no inclusion into or onto article) |
| ERC7 | Use of functional fluid at industrial site |
| ERC8d | Widespread use of non-reactive processing aid (no inclusion into or onto article, outdoor) |
| ERC9a | Widespread use of functional fluid (indoor) |
| ERC9b | Widespread use of functional fluid (outdoor) |

| Product (article) characteristics |  |
| --- | --- |
| Physical form of product | See section 9 of the SDS, No additional information |
| Concentration of substance in product | ≤ 100 % |

| Amount used, frequency and duration of use (or from service life) |  |
| --- | --- |
| The actual tonnage handled per site is not considered to influence the immissions as such for this scenario as there is practically no release |  |
| Emission Days (days/year) | 260 |

| Technical and organisational conditions and measures |  |
| --- | --- |
| Ensure operatives are trained to minimise releases |  |

| Conditions and measures related to sewage treatment plant |  |
| --- | --- |
| Wastewater emission controls are not applicable as there is no direct release to wastewater |  |

| Conditions and measures related to treatment of waste (including article waste) |  |
| --- | --- |
| See section 13 of the SDS |  |

| Other conditions affecting environmental exposure |  |
| --- | --- |
| No additional information |  |

1.2.2. Control of worker exposure: Chemical production or refinery in closed process without likelihood of exposure or processes with equivalent containment conditions, Chemical production or refinery in closed continuous process with occasional controlled exposure or processes with equivalent containment conditions, Manufacture or formulation in the chemical industry in closed batch processes with occasional controlled exposure or processes with equivalent containment condition, Transfer of substance or mixture (charging and discharging) at dedicated facilities, Transfer of substance or mixture into small containers (dedicated filling line, including weighing), Use of fuels (PROC1, PROC2, PROC3, PROC8b, PROC9, PROC16)

| PROC1 | Chemical production or refinery in closed process without likelihood of exposure or processes with equivalent containment conditions |
| --- | --- |
| PROC2 | Chemical production or refinery in closed continuous process with occasional controlled exposure or processes with equivalent containment conditions |
| PROC3 | Manufacture or formulation in the chemical industry in closed batch processes with occasional controlled exposure or processes with equivalent containment condition |
| PROC8b | Transfer of substance or mixture (charging and discharging) at dedicated facilities |
| PROC9 | Transfer of substance or preparation into small containers (dedicated filling line, including weighing) |
| PROC16 | Use of fuels |

| Product (article) characteristics |  |
| --- | --- |
| Physical form of product | See section 9 of the SDS, No additional information |
| Concentration of substance in product | ≤ 100 % |

| Amount used (or contained in articles), frequency and duration of use/exposure |  |
| --- | --- |
| The actual tonnage handled per shift is not considered to influence the exposure as such for this scenario. Instead, the combination of the scale of operation and level of containment/automation (as reflected in the technical conditions) is the main determinant of the process-intrinsic emission potential. |  |
| Exposure duration | ≤ 8 h/day |
| Covers frequency up to: | 5 days/week |

| Technical and organisational conditions and measures |  |
| --- | --- |
| See sections 2 and 7 of the SDS. |  |
| Handle product within a closed system |  |
| Apply a good standard of general or controlled ventilation when maintenance activities are carried out. |  |
| Ensure operatives are trained to minimise exposure |  |
| Ensure supervision is in place to check that the RMMs are in place and are being used correctly and that the OCs are being followed |  |

| Conditions and measures related to personal protection, hygiene and health evaluation |  |
| --- | --- |
| See section 8 of the SDS. |  |

| Other conditions affecting workers exposure |  |
| --- | --- |
| Indoor or outdoor use |  |

### 1.3. Exposure estimation and reference to its source

No data available

| 1.4. Guidance to Downstream User to evaluate whether he works inside the boundaries set by the ES |  |
| --- | --- |
| 1.4.1. Environment |  |
| Guidance - Environment | Check that RMMs and OCs are as described above or of equivalent efficiency |
| 1.4.2. Health |  |
| Guidance - Health | Check that RMMs and OCs are as described above or of equivalent efficiency |

## 2. EIGA001-2 - Professional uses

### 2.1. Title section

| Professional uses |  |
| --- | --- |
| ES Ref.: EIGA001-2 ES Type: Worker - EIGA Revision date: 2-12-2019 |  |

| Processes, tasks, activities covered | Professional uses, including transfer of product in non-industrial settings |
| --- | --- |
| Assessment method | ECETOC TRA 2.0 |
| 2.2. Conditions of use affecting exposure |  |

2.2.1. Control of environmental exposure: Widespread use of functional fluid (indoor), Widespread use of functional fluid (outdoor) (ERC9a, ERC9b)

| ERC9a | Widespread use of functional fluid (indoor) |
| --- | --- |
| ERC9b | Widespread use of functional fluid (outdoor) |

| Product (article) characteristics |  |
| --- | --- |
| Physical form of product | See section 9 of the SDS, No additional information |
| Concentration of substance in product | ≤ 100 % |

| Amount used, frequency and duration of use (or from service life) |  |
| --- | --- |
| No additional information |  |

| Technical and organisational conditions and measures |  |
| --- | --- |
| Ensure operatives are trained to minimise exposure |  |

| Conditions and measures related to sewage treatment plant |  |
| --- | --- |
| No additional information |  |

| Conditions and measures related to treatment of waste (including article waste) |  |
| --- | --- |
| See section 13 of the SDS |  |

| Other conditions affecting environmental exposure |  |  |
| --- | --- | --- |
| Closed systems are used in order to prevent unintended emissions |  |  |

2.2.2. Control of worker exposure: Use of fuels (PROC16)

| PROC16 | Use of fuels |
| --- | --- |

| Product (article) characteristics |  |
| --- | --- |
| Physical form of product | See section 9 of the SDS, No additional information |
| Concentration of substance in product | ≤ 100 % |

| Amount used (or contained in articles), frequency and duration of use/exposure |  |
| --- | --- |
| The actual tonnage handled per shift is not considered to influence the exposure as such for this scenario. Instead, the combination of the scale of operation and level of containment/automation (as reflected in the technical conditions) is the main determinant of the process-intrinsic emission potential. |  |
| Exposure duration | ≤ 8 h/day |
| Covers frequency up to: | 5 days/week |

| Technical and organisational conditions and measures |  |
| --- | --- |
| Handle product within a closed system |  |
| Apply a good standard of general or controlled ventilation when maintenance activities are carried out. |  |
| See sections 2 and 7 of the SDS. |  |
| Ensure operatives are trained to minimise exposure |  |
| Ensure supervision is in place to check that the RMMs are in place and are being used correctly and that the OCs are being followed |  |

| Conditions and measures related to personal protection, hygiene and health evaluation |  |
| --- | --- |
| See section 8 of the SDS. |  |

| Other conditions affecting workers exposure |  |
| --- | --- |
| Indoor or outdoor use |  |

### 2.3. Exposure estimation and reference to its source

No data available

| 2.4. Guidance to Downstream User to evaluate whether he works inside the boundaries set by the ES |  |
| --- | --- |
| 2.4.1. Environment |  |
| Guidance - Environment | Check that RMMs and OCs are as described above or of equivalent efficiency |
| 2.4.2. Health |  |
| Guidance - Health | Check that RMMs and OCs are as described above or of equivalent efficiency |

3. EIGA001-3 - Consumer use.

### 3.1. Title section

| Consumer use. |  |
| --- | --- |
| ES Ref.: EIGA001-3 ES Type: Consumer - EIGA Revision date: 2-12-2019 |  |

| Processes, tasks, activities covered | Consumer use. Use as a fuel. |
| --- | --- |
| 3.2. Conditions of use affecting exposure |  |

3.2.1. Control of environmental exposure: Widespread use of functional fluid (indoor). Widespread use of functional fluid (outdoor). (ERC9a, ERC9b)

| ERC9a | Widespread use of functional fluid (indoor) |
| --- | --- |
| ERC9b | Widespread use of functional fluid (outdoor) |

| Product (article) characteristics |  |
| --- | --- |
| Physical form of product | See section 9 of the SDS, No additional information |
| Concentration of substance in product | ≤ 100 % |

| Amount used, frequency and duration of use (or from service life) |  |
| --- | --- |
| No additional information |  |

| Conditions and measures related to treatment of waste (including article waste) |  |
| --- | --- |
| See section 13 of the SDS |  |

| Other conditions affecting environmental exposure |  |  |
| --- | --- | --- |
| Closed systems are used in order to prevent unintended emissions |  |  |

3.2.2. Control of consumer exposure:  (PC13)

| PC13 | Fuels |
| --- | --- |

| Product (article) characteristics |  |
| --- | --- |
| Physical form of product | See section 9 of the SDS, No additional information |
| Concentration of substance in product | ≤ 100 % |

| Amount used (or contained in articles), frequency and duration of use/exposure |  |
| --- | --- |
| The actual tonnage handled per shift is not considered to influence the exposure as such for this scenario. Instead, the combination of the scale of operation and level of containment/automation (as reflected in the technical conditions) is the main determinant of the process-intrinsic emission potential. |  |
| Exposure duration | ≤ 8 h/day |
| Amount used (or contained in articles), frequency and duration of use/exposure |  |
| Covers frequency up to: | 5 days/week |

| Measures related to information and behavioural advice to consumers including personal protection and hygiene |  |
| --- | --- |
| See section 8 of the SDS. |  |

| Other conditions affecting consumer exposure |  |
| --- | --- |
| Indoor or outdoor use |  |

### 3.3. Exposure estimation and reference to its source

No data available

| 3.4. Guidance to Downstream User to evaluate whether he works inside the boundaries set by the ES |  |
| --- | --- |
| 3.4.1. Environment |  |
| Guidance - Environment | Check that RMMs and OCs are as described above or of equivalent efficiency |
| 3.4.2. Health |  |
| Guidance - Health | Check that RMMs and OCs are as described above or of equivalent efficiency |

End of document