|  | SAFETY DATA SHEET |  |
| --- | --- | --- |
|  |  | Doc. No.: SDS – AL007B |
|  |  | Page: Page 1 of 6 Owner: SHEQ |
|  |  | Rev. Date: September, 2022 |
| Nitrogen (Liquid) |  |  |

| 1 IDENTIFICATION OF THE SUBSTANCE / PREPARATION AND OF THE COMPANY / UNDERTAKING |  |  |
| --- | --- | --- |
| Trade name MSDS No. Chemical Formula Company Identification |  | : Nitrogen (Liquid) : AL007B : N2 : Janox BV Roggestraat 12 8972 Krombeke +32(0) 57 30 98 40 info@janox.be |
| 2 HAZARDS IDENTIFICATION |  |  |

Classification of the substance

- Physical Hazard 	 	 	 	: Gases under pressure  - Refrigerated liquefied gas

- Health Hazard  	 	 	 	: no acute toxicity

Label Elements

- Hazard Pictogram

- Signal word 	 	 	 	: Warning

|  | SAFETY DATA SHEET |  |
| --- | --- | --- |
|  |  | Doc. No.: SDS – AL007B |
|  |  | Page: Page 2 of 6 Owner: SHEQ |
|  |  | Rev. Date: September, 2022 |
| Nitrogen (Liquid) |  |  |

# 2 HAZARDS IDENTIFICATION (continued 	)

- Hazard Statement  	 	 	: Contains refrigerated gas; may cause cryogenic burns or

injury

- Precautionary statements o Prevention    : Wear cold insulating gloves / face shield / eye protection

  - Response  	 	 	: Thaw frosted parts with lukewarm water. Do not rub affected

area

: Get immediate medical advice / attention

  - Storage 	 	 	 	: Store in a well ventilated area

Other Hazard  	Other hazards 	 	 	 	: Contact with product may cause cold burns or frostbite.

In high concentration may cause asphyxiation.

# 3 COMPOSITION / INFORMATION ON INGREDIENTS

Substance / Preparation 	 	 	: Substance

Substance Name Contents CAS No. EC No.  Index No. Classification  Nitrogen (refrigerated)   100%  7727-37-9 231-783-9  --------

Contains no other components or impurities which will influence the classification of the product.

# 4 FIRST AID MEASURES

| First Aid Measures |  |  |
| --- | --- | --- |
| - Inhalation |  | : In high concentrations may cause asphyxiation. Symptoms may include loss of mobility / consciousness. Victim may not be aware of asphyxiation Remove victim to uncontaminated area wearing self contained breathing apparatus. Keep victim warm and rested. Call a doctor. Apply artificial respiration if breathing stopped. |
| - Skin / Eye Contact |  | : Immediately flush eyes thoroughly with water for at least 15 minutes. In case of frostbite spray with water for at least 15 minutes. Apply a sterile dressing. Obtain medical assistance. |
| - Ingestion |  | : Ingestion is not considered a potential route of exposure. |

|  | SAFETY DATA SHEET |  |
| --- | --- | --- |
|  |  | Doc. No.: SDS – AL007B |
|  |  | Page: Page 3 of 6 Owner: SHEQ |
|  |  | Rev. Date: S eptember, 2022 |
| Nitrogen (Liquid) |  |  |

| 5 FIRE FIGHTING MEASURES |  |
| --- | --- |
| Flammable class Specific hazards Hazardous Combustion Products Extinguishing media - Suitable extinguishing media Specific methods Special protective equipment for fire fighters | : Non flammable : Exposure to fire may cause containers to rupture / explode : None : All known extinguishants can be used. : If possible stop flow of product. Move away from the container and cool with water from a protected position. If leaking do not spray water onto container. Water surrounding area (from protected position) to contain fire. : In confined space use self-contained breathing apparatus |
| 6 ACCIDENTAL RELEASE MEASURES |  |
| Personal precautions Environmental Precautions Clean up methods | : Evacuate area Use protective clothing Wear self-contained breathing apparatus when entering area unless atmosphere is proved to be safe. Ensure adequate air ventilation. : Try to stop release. Prevent from entering sewers, basements and workpits, or any place where its accumulation can be dangerous. : Ventilate area. |
| 7 HANDLING AND STORAGE |  |

Storage 	: Keep container below 50 degrees C in a well ventilated place.

Handling 	: Suck back of water into the container must be prevented  	 	 	 	 	 	 	Do not allow backfeed into the container.

Use only properly specified equipment which is suitable for this product, its supply pressure and temperature. Contact your gas supplier if in doubt.

Refer to supplier’s container handling instructions.

|  | SAFETY DATA SHEET |  |
| --- | --- | --- |
|  |  | Doc. No.: SDS – AL007B |
|  |  | Page: Page 4 of 6 Owner: SHEQ |
|  |  | Rev. Date: S eptember, 2022 |
| Nitrogen (Liquid) |  |  |

| 8 EXPOSURE CONTROLS / PERSONAL PROTECTION |  |
| --- | --- |
| Personal protection | : Ensure adequate ventilation. Protect eyes, face and skin from liquid splashes. |
| 9 PHYSICAL AND CHEMICAL PROPERTIES |  |
| Physical state at 20 degrees C Color Odor Molecular Weight Melting point (degrees C) Boiling point (degrees C) Critical temperature (degrees C) Vapor pressure, 20 degrees C Relative density, gas (air =1) Relative density, liquid (water = 1) Solubility in water [mg/l] Other data | : Refrigerated liquefied gas. : Colorless liquid : No odor warning properties : 28 : -210 : -196 : -147 : Not Applicable : 0.97 : 0.8 : 20 : Gas/vapor heavier than air. May accumulate in confined spaces, particularly at or below ground level. |
| 10 STABILITY AND REACTIVITY |  |

Stability and reactivity  	 	 	: Stable under normal conditions.

Liquid spillages can cause embrittlement of structural materials.

|  | SAFETY DATA SHEET | Doc. No.: SDS – AL007B Page: Page 5 of 6 Owner: SHEQ Rev. Date: S eptember, 2022 |
| --- | --- | --- |
| Nitrogen (Liquid) |  |  |

11 TOXICOLOGICAL INFORMATION

| Toxicity information | : No known toxicological effects from this product |
| --- | --- |
| 12 ECOLOGICAL INFORMATION |  |
| Ecological effects information | : Can cause frost damage to vegetation. |
| 13 DISPOSAL CONSIDERATIONS |  |
| General | : Do not discharge into any place where its accumulation could be dangerous. Contact supplier if guidance is required. |
| 14 TRANSPORT INFORMATION |  |

UN No.  	 	 	 	 	: 1977

H.I. nr  	 	 	 	 	: 22

ADR / RID

- Proper shipping name 	 	: Nitrogen, Refrigerated Liquid

- ADR Class 	 	 	 	: 2

ADR / RID Classification code 	: 3 A

- Labelling ADR 	 	 	:

2.2: Non flammable, non toxic gas.

Other transport information : Avoid transport on vehicles where the load space is not separated from the driver’s compartment

Ensure vehicle driver is aware of the potential hazards of the

load and knows what to do in the event of an accident or an emergency.

Before transporting product containers:

- Ensure that containers are firmly secured.

- Ensure cylinder valve is closed and not leaking.

- Ensure valve outlet cap nut or plug (where provided) is correctly fitted.

|  | SAFETY DATA SHEET |  |
| --- | --- | --- |
|  |  | Doc. No.: SDS – AL007B |
|  |  | Page: Page 6 of 6 Owner: SHEQ |
|  |  | Rev. Date: S eptember, 2022 |
| Nitrogen (Liquid) |  |  |

| 14 TRANSPORT INFORMATION (continued) | - - |
| --- | --- |

- Ensure valve protection device (where provided) is correctly fitted.

- Ensure there is adequate ventilation.

- Compliance with applicable regulations

| 15 REGULATORY INFORMATION |  |
| --- | --- |
| EC Classification EC Labeling - Symbol (s) - R Phrase (s) - S Phrase (s) | : Not included in Annex I. Not classified as dangerous preparation / substance : No EC labeling required. : None : None : None |
| 16 OTHER INFORMATION |  |

- May cause frostbite.

- Keep container in a well-ventilated place.

- Do not breathe the gas.

- Ensure all national / local regulations are observed.

- Asphyxiant in high concentrations.

- The hazard of asphyxiation is often overlooked and must be stressed during operator training.

- This Material Safety Data Sheet has been established in accordance with the applicable European Directives and applies to all countries that have translated the Directives in their national laws.

- Before using this product in any new process or experiment, a thorough material compatibility and safety study should be carried out.

- Details given in this document are believed to be correct at the time of going to press. Whilst proper care has been taken in the preparation of this document, no liability for injury or damage resulting from its use can be accepted.

Recommended uses and restrictions: This MSDS is for information purposes only and is subject to change without prior notice. [Prior to purchase of products, please contact your local AIR LIQUIDE office for complete MSDS (with Manufacturer’s name and emergency phone number)].

End of Document