Carbon dioxide (solid) – Dry ice

According to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830

Date of issue: 01/04/2015 Supersedes: 01/03/2018 Revision date: 08/07/2021 Version: 6.0 Reference number: EIGA018C

# SECTION 1: Identification of the substance/mixture and of the company/undertaking

## 1.1. Product identifier

| Trade name | : Carbon dioxide (solid) – dry ice |
| --- | --- |
| SDS no | : EIGA018C |
| Chemical description | : Carbon dioxide (solid) |
|  | CAS-No. : 124-38-9 |
|  | EC-No. : 204-696-9 |
|  | EC Index-No. : --- |
| REACH registration No | : Listed in Annex IV / V REACH, exempted from registration. |
| Chemical formula | : CO2 |

## 1.2. Relevant identified uses of the substance or mixture and uses advised against

Relevant identified uses 	: Industrial and professional uses. Perform risk assessment prior to use.

Cooling (Food additive E290).

Blast cleaning.

Metal cooling.

Contact supplier for more information on uses.

Consumer use.

Uses advised against 	: In beverage for fogging effect, because of the risk of ingestion.

## 1.3. Details of the supplier of the safety data sheet

Company identification 	: Janox BV

Roggestraat 12

8972 Krombeke

+32(0)57 30 98 40

Belguim

info@janox.be

1.4. Emergency telephone number

Emergency telephone number 	: Tel : +32 57 30 98 40

# SECTION 2: Hazards identification

## 2.1. Classification of the substance or mixture

Classification according to Regulation (EC) No. 1272/2008 [CLP] Not regulated.

## 2.2. Label elements

Labelling according to Regulation (EC) No. 1272/2008 [CLP]

| Precautionary statements (CLP) |  |
| --- | --- |
| - Storage 2.3. Other hazards | : P403 - Store in a well-ventilated place. |
|  | Refrigerated solidified gas. Contact with product may cause cold burns or frostbite. In high concentrations CO2 causes rapid circulatory insufficiency even at normal levels of oxygen concentration. Symptoms are headache, nausea and vomiting, which may lead to unconsciousness and death. |

# SECTION 3: Composition/information on ingredients

## 3.1. Substances

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| Carbon dioxide (solid) | CAS-No.: 124-38-9 EC-No.: 204-696-9 EC Index-No.: --- REACH registration No: *1 | 100 | Not classified |

Contains no other components or impurities which will influence the classification of the product.

*1: Listed in Annex IV / V REACH, exempted from registration.

*3: Registration not required: Substance manufactured or imported < 1t/y.

3.2. Mixtures 	 Not applicable

# SECTION 4: First aid measures

## 4.1. Description of first aid measures

- Inhalation 	: Adverse effects not expected from this product.

- Skin contact 	: In case of frostbite spray with water for at least 15 minutes. Apply a sterile dressing. Obtain medical assistance.

- Eye contact : Immediately flush eyes thoroughly with water for at least 15 minutes. - Ingestion : Get immediate medical attention.

## 4.2. Most important symptoms and effects, both acute and delayed

Low concentrations of CO2 cause increased respiration and headache.

Refer to section 11.

4.3. Indication of any immediate medical attention and special treatment needed

None.

# SECTION 5: Firefighting measures

## 5.1. Extinguishing media

- Suitable extinguishing media 	: Water spray or fog.

Not applicable.

Product does not burn, use fire control measures appropriate for the surrounding fire.

- Unsuitable extinguishing media 	: Not applicable.

Do not use water jet to extinguish.

## 5.2. Special hazards arising from the substance or mixture

| Specific hazards | : Exposure to fire may cause containers to rupture/explode. |
| --- | --- |
| Hazardous combustion products 5.3. Advice for firefighters | : None. |
| Specific methods | : Use fire control measures appropriate for the surrounding fire. Exposure to fire and heat radiation may cause gas receptacles to rupture. Cool endangered receptacles with water spray jet from a protected position. Prevent water used in emergency cases from entering sewers and drainage systems. If possible, stop flow of product. Use water spray or fog to knock down fire fumes if possible. Move containers away from the fire area if this can be done without risk. |

# SECTION 6: Accidental release measures

## 6.1. Personal precautions, protective equipment and emergency procedures

| For non-emergency personnel | : Act in accordance with local emergency plan. Prevent from entering sewers, basements and workpits, or any place where its accumulation can be dangerous. Stay upwind. See section 8 of the SDS for more information on personal protective equipment |
| --- | --- |
| For emergency responders | : See section 5.3 of the SDS for more information. |

6.2. Environmental precautions

No additional information available

6.3. Methods and material for containment and cleaning up

Ventilate area.

6.4. Reference to other sections

See also sections 8 and 13.

# SECTION 7: Handling and storage

## 7.1. Precautions for safe handling

Safe use of the product 	: For more guidance on safe use, refer to the EIGA Doc.150 “Code of practice Dry Ice” downloadable at http://www.eiga.eu. and consult your supplier.

The product must be handled in accordance with good industrial hygiene and safety procedures.

Only experienced and properly instructed persons should handle  gases under pressure.

Consider pressure relief device(s) in gas installations.

Ensure the complete gas system was (or is regularily) checked for leaks before use.

Do not smoke while handling product.

Use only properly specified equipment which is suitable for this product, its supply pressure and temperature. Contact your gas supplier if in doubt.

Avoid suck back of water, acid and alkalis.

Do not breathe gas.

Avoid release of product into atmosphere.

Safe handling of the gas receptacle 	: Refer to supplier's container handling instructions.

Do not allow backfeed into the container.

Protect containers from physical damage; do not drag, roll, slide or drop.

When moving cylinders, even for short distances, use a cart (trolley, hand truck, etc.) designed to transport cylinders.

Leave valve protection caps in place until the container has been secured against either a wall or bench or placed in a container stand and is ready for use.

If user experiences any difficulty operating valve discontinue use and contact supplier.

Never attempt to repair or modify container valves or safety relief devices.

Damaged valves should be reported immediately to the supplier.

Keep container valve outlets clean and free from contaminants particularly oil and water. Replace valve outlet caps or plugs and container caps where supplied as soon as container is disconnected from equipment.

Close container valve after each use and when empty, even if still connected to equipment.

Never attempt to transfer gases from one cylinder/container to another.

Never use direct flame or electrical heating devices to raise the pressure of a container. Do not remove or deface labels provided by the supplier for the identification of the content of the container.

Suck back of water into the container must be prevented.

Open valve slowly to avoid pressure shock.

## 7.2. Conditions for safe storage, including any incompatibilities

Observe all regulations and local requirements regarding storage of containers.

Containers should not be stored in conditions likely to encourage corrosion.

Container valve guards or caps should be in place.

Containers should be stored in the vertical position and properly secured to prevent them from falling over.

Stored containers should be periodically checked for general condition and leakage.

Keep container below 50°C in a well ventilated place.

Store containers in location free from fire risk and away from sources of heat and ignition. Keep away from combustible materials.

7.3. Specific end use(s)

None.

# SECTION 8: Exposure controls/personal protection

## 8.1. Control parameters

| Carbon dioxide (solid) (124-38-9) |  |
| --- | --- |
| EU - Indicative Occupational Exposure Limit (IOEL) |  |
| Local name | Carbon dioxide |
| IOEL TWA | 9000 mg/m³ |
| IOEL TWA [ppm] | 5000 ppm |
| Regulatory reference | COMMISSION DIRECTIVE 2006/15/EC |
| Albania - Occupational Exposure Limits |  |
| Local name | Dioksid karboni |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Regulatory reference | VENDIM Nr. 522, datë 6.8.2014 PËR MIRATIMIN E RREGULLORES “PËR MBROJTJEN E SIGURISË DHE SHËNDETIT TË PUNËMARRËSVE NGA RISQET E LIDHURA ME AGJENTËT KIMIKË NË PUNË” |
| Belgium - Occupational Exposure Limits |  |
| Local name | Carbone (dioxyde de) # Koolstofdioxide |
| OEL TWA | 9131 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| OEL STEL | 54784 mg/m³ |
| OEL STEL [ppm] | 30000 ppm |
| Remark (BE) | A: la mention “A” signifie que l’agent libère un gaz ou une vapeur qui n’ont en eux-mêmes aucun effet physiologique mais peuvent diminuer le taux d’oxygène dans l’air. Lorsque le taux d’oxygène descend en dessous de 1718 % (vol/vol) le manque d’oxygène provoque des suffocations qu’aucun symptôme préalable n’annonce. # A: de vermelding “A” betekent dat dit agens gas of damp vrijgeeft dat of die op zich geen fysiologische werking heeft, maar het zuurstofgehalte in de lucht verlaagt. Wanneer het zuurstofgehalte daalt onder de 17-18 % (vol/vol), veroorzaakt het zuurstoftekort verstikking, die zich manifesteert zonder dat er een waarschuwing aan voorafgaat. |

| Regulatory reference | Koninklijk besluit/Arrêté royal 21/01/2020 |
| --- | --- |
| Bulgaria - Occupational Exposure Limits |  |
| Local name | Въглероден диоксид |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Notes | • (Химични агенти, за които са определени гранични стойности във въздуха на работната среда за Европейската общност) |
| Regulatory reference | Наредба № 13 от 30.12.2003 г. за защита на работещите от рискове, свързани с експозиция на химични агенти при работа (изм. и доп. ДВ. бр.5 от 17 Януари 2020 г.) |
| Croatia - Occupational Exposure Limits |  |
| Local name | Ugljikov dioksid |
| GVI (OEL TWA) [1] | 9000 mg/m³ |
| GVI (OEL TWA) [2] | 5000 ppm |
| Naznake (HR) | Direktiva: 2006/15/EZ |
| Regulatory reference | Pravilnik o izmjenama i dopunama Pravilnika o graničnim vrijednostima izloženosti opasnim tvarima pri radu i o biološkim graničnim vrijednostima (NN 91/2018) |
| Cyprus - Occupational Exposure Limits |  |
| Local name | Διοξείδιο του άνθρακα |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Regulatory reference | Κανονισμοί του 2007 (Κ.Δ.Π. 295/2007) |
| Czech Republic - Occupational Exposure Limits |  |
| Local name | Oxid uhličitý |
| PEL (OEL TWA) | 9000 mg/m³ |
| PEL (OEL TWA) [ppm] | 4923 ppm |
| NPK-P (OEL C) | 45000 mg/m³ |
| NPK-P (OEL C) [ppm] | 24615 ppm |
| Regulatory reference | Nařízení vlády č. 361/2007 Sb. (Předpis 41/2020 Sb.) |
| Denmark - Occupational Exposure Limits |  |
| Local name | Carbondioxid (Kuldioxid; Kulsyre) |
| OEL TWA [1] | 9000 mg/m³ |
| OEL TWA [2] | 5000 ppm |
| Anmærkninger (DK) | E (betyder, at stoffet har en EF-grænseværdi) |
| Regulatory reference | BEK nr 1458 af 13/12/2019 |
| Estonia - Occupational Exposure Limits |  |
| Local name | Süsinikdioksiid |
| OEL TWA | 9000 mg/m³ |

| OEL TWA [ppm] | 5000 ppm |
| --- | --- |
| Remark (ET) | 8 (Süsinikdioksiid on õhu saastatuse indikaatoriks töökohtadel, kus õhk saastub töötajate suure füüsilise aktiivsuse tõttu) |
| Regulatory reference | Vabariigi Valitsuse 20. märtsi 2001. a määruse nr 105 (RT I, 17.10.2019, 2); Vabariigi Valitsuse 10. märtsi 2019. a määruse nr 84 |
| Finland - Occupational Exposure Limits |  |
| Local name | Hiilidioksidi |
| HTP (OEL TWA) [1] | 9100 mg/m³ |
| HTP (OEL TWA) [2] | 5000 ppm |
| Regulatory reference | HTP-ARVOT 2018 (Sosiaali- ja terveysministeriö) |
| France - Occupational Exposure Limits |  |
| Local name | Carbone (dioxyde de) |
| VME (OEL TWA) | 9000 mg/m³ |
| VME (OEL TWA) [ppm] | 5000 ppm |
| Note (FR) | Valeurs règlementaires indicatives |
| Regulatory reference | Arrêté du 30 juin 2004 modifié (réf.: INRS ED 984, 2016) |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| Local name | Kohlenstoffdioxid |
| AGW (OEL TWA) [1] | 9100 mg/m³ |
| AGW (OEL TWA) [2] | 5000 ppm |
| Peak exposure limitation factor | 2(II) |
| Remark | DFG;EU |
| Regulatory reference | TRGS900 |
| Gibraltar - Occupational Exposure Limits |  |
| Name of agent | Carbon dioxide |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Regulatory reference | Factories (Control of Chemical Agents at Work) Regulations 2003 (LN. 2018/181) |
| Greece - Occupational Exposure Limits |  |
| Local name | Διοξείδιο του άνθρακα |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| OEL STEL | 54000 mg/m³ |
| Regulatory reference | Π.Δ. 90/1999 - Προστασία της υγείας των εργαζομένων που εκτίθενται σε ορισμένους χημικούς παράγοντες κατά τη διάρκεια της εργασίας τους |
| Hungary - Occupational Exposure Limits |  |
| Local name | SZÉN-DIOXID |

| AK (OEL TWA) | 9000 mg/m³ |
| --- | --- |
| Megjegyzések (HU) | EU2 (2006/15/EK irányelvben közölt érték); N (Irritáló anyagok, egyszerű fojtógázok, csekély egészségkárosító hatással bíró anyagok) |
| Regulatory reference | 5/2020. (II. 6.) ITM rendelet - A kémiai kóroki tényezők hatásának kitett munkavállalók egészségének és biztonságának védelméről |
| Ireland - Occupational Exposure Limits |  |
| Local name | Carbon dioxide |
| OEL TWA [1] | 9000 mg/m³ |
| OEL TWA [2] | 5000 ppm |
| Notes (IE) | IOELV (Indicative Occupational Exposure Limit Values) |
| Regulatory reference | Chemical Agents Code of Practice 2020 |
| Italy - Occupational Exposure Limits |  |
| Local name | Anidride carbonica |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Regulatory reference | Allegato XXXVIII del D.Lgs. 9 aprile 2008, n. 81 e s.m.i. |
| Latvia - Occupational Exposure Limits |  |
| Local name | Oglekļa dioksīds |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Regulatory reference | Ministru kabineta 2007. gada 15. maija noteikumiem Nr. 325 (Grozījumi Ministru kabineta 2011. gada 1. februārī noteikumiem Nr. 92) |
| Lithuania - Occupational Exposure Limits |  |
| Local name | Anglies dioksidas |
| IPRV (OEL TWA) | 9000 mg/m³ |
| IPRV (OEL TWA) [ppm] | 5000 ppm |
| Remark (LT) | Anglies dioksidas dažnai laikomas kaip indikatorius darbo patalpose, kuriose oro teršalai susidaro dėl žmonių buvimo jose. |
| Regulatory reference | LIETUVOS HIGIENOS NORMA HN 23:2011 (Nr. V-695/A1-272, 2018-06- 12) |
| Luxembourg - Occupational Exposure Limits |  |
| Local name | Dioxyde de carbone |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Regulatory reference | Mémorial A Nº 684 de 2018 concernant la protection de la sécurité et de la santé des salariés contre les risques liés à des agents chimiques sur le lieu de travail |
| Malta - Occupational Exposure Limits |  |
| Local name | Carbon dioxide |
| OEL TWA | 9000 mg/m³ |

| OEL TWA [ppm] | 5000 ppm |
| --- | --- |
| Regulatory reference | S.L.424.24 - Chemical Agents at Work Regulations (L.N.57 of 2018) |
| Netherlands - Occupational Exposure Limits |  |
| Local name | Kooldioxide |
| MAC-TGG (OEL TWA) | 9000 mg/m³ |
| Regulatory reference | Arbeidsomstandighedenregeling 2020 |
| Poland - Occupational Exposure Limits |  |
| Local name | Ditlenek węgla |
| NDS (OEL TWA) | 9000 mg/m³ |
| NDSCh (OEL STEL) | 27000 mg/m³ |
| Regulatory reference | Dz. U. 2018 poz. 1286 |
| Portugal - Occupational Exposure Limits |  |
| Local name | Dióxido de carbono |
| OEL TWA [ppm] | 5000 ppm |
| OEL STEL [ppm] | 30000 ppm |
| Regulatory reference | Norma Portuguesa NP 1796:2014 |
| Romania - Occupational Exposure Limits |  |
| Local name | Dioxid de carbon |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Regulatory reference | Hotărârea Guvernului nr. 1.218/2006 (Hotărârea nr. 157/2020) |
| Serbia - Occupational Exposure Limits |  |
| Local name | угљен-диоксид |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Notes | ЕU** – напомена да се ради о хемијским материјама за које су утврђене индикативне граничне вредности изложености према Директиви 2006/15/ЕЗ (друга листа) |
| Regulatory reference | ПРАВИЛНИК о превентивним мерама за безбедан и здрав рад при излагању хемијским материјама (,,Службени гласник РС”, бр. 106/09 и 117/17) |
| Slovakia - Occupational Exposure Limits |  |
| Local name | Oxid uhličitý |
| NPHV (OEL TWA) [1] | 9000 mg/m³ |
| NPHV (OEL TWA) [2] | 5000 ppm |
| Regulatory reference | Nariadenie vlády č. 33/2018 Z. z. |
| Slovenia - Occupational Exposure Limits |  |
| Local name | ogljikov dioksid |

| OEL TWA | 9000 mg/m³ |
| --- | --- |
| OEL TWA [ppm] | 5000 ppm |
| OEL STEL | 18000 mg/m³ |
| OEL STEL [ppm] | 10000 ppm |
| Remark (SI) | EU |
| Regulatory reference | Uradni list RS, št. 78/2019 z dne 20.12.2019 |
| Spain - Occupational Exposure Limits |  |
| Local name | Dióxido de carbono |
| VLA-ED (OEL TWA) [1] | 9150 mg/m³ |
| VLA-ED (OEL TWA) [2] | 5000 ppm |
| Notes | VLI (Agente químico para el que la U.E. estableció en su día un valor límite indicativo). |
| Regulatory reference | Límites de Exposición Profesional para Agentes Químicos en España 2019. INSHT |
| Sweden - Occupational Exposure Limits |  |
| Local name | Koldioxid |
| NGV (OEL TWA) | 9000 mg/m³ |
| NGV (OEL TWA) [ppm] | 5000 ppm |
| KTV (OEL STEL) | 18000 mg/m³ |
| KTV (OEL STEL) [ppm] | 10000 ppm |
| Anmärkning (SE) | V (Vägledande korttidsgränsvärde ska användas som ett rekommenderat högsta värde som inte bör överskridas); 34 (Koldioxid används ofta som indikatorsubstans i arbetslokaler där luftföroreningar huvudsakligen uppkommer genom de personer som vistas där) |
| Regulatory reference | Hygieniska gränsvärden (AFS 2018:1) |
| United Kingdom - Occupational Exposure Limits |  |
| Local name | Carbon dioxide |
| WEL TWA (OEL TWA) [1] | 9150 mg/m³ |
| WEL TWA (OEL TWA) [2] | 5000 ppm |
| WEL STEL (OEL STEL) | 27400 mg/m³ |
| WEL STEL (OEL STEL) [ppm] | 15000 ppm |
| Regulatory reference | EH40/2005 (Fourth edition, 2020). HSE |
| Iceland - Occupational Exposure Limits |  |
| Local name | Koldíoxíð (koltvísýringur, kolsýra) |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Regulatory reference | Reglugerð um mengunarmörk og aðgerðir til að draga úr mengun á vinnustöðum (Nr. 390/2009) |
| Norway - Occupational Exposure Limits |  |
| Local name | Karbondioksid |
| Grenseverdi (OEL TWA) [1] | 9000 mg/m³ |
| Grenseverdi (OEL TWA) [2] | 5000 ppm |
| Merknader (NO) | E: EU har en veiledende grenseverdi for stoffet. |
| Regulatory reference | FOR-2020-04-06-695 |
| Switzerland - Occupational Exposure Limits |  |
| Local name | Gaz carbonique / Kohlendioxid [Kohlenstoffdioxid] |
| MAK (OEL TWA) [1] | 9000 mg/m³ |
| MAK (OEL TWA) [2] | 5000 ppm |
| Critical toxicity | Asphyxie / Asphyxie |
| Remark | NIOSH |
| Regulatory reference | www.suva.ch, 01.01.2020 |
| Turkey - Occupational Exposure Limits |  |
| Local name | Karbondioksit |
| OEL TWA | 9000 mg/m³ |
| OEL TWA [ppm] | 5000 ppm |
| Regulatory reference | 12 Ağustos 2013 Tarihli ve 28733 Sayılı Resmî Gazete |

| DNEL (Derived-No Effect Level) | : None available. |
| --- | --- |
| PNEC (Predicted No-Effect Concentration) 8.2. Exposure controls 8.2.1. Appropriate engineering controls | : None available. |
|  | Systems under pressure should be regularily checked for leakages. Ensure exposure is below occupational exposure limits (where available). Consider the use of a work permit system e.g. for maintenance activities. CO2 detectors should be used when CO2 may be released. |

8.2.2. Individual protection measures, e.g. personal protective equipment

A risk assessment should be conducted and documented in each work area to assess the

risks related to the use of the product and to select the PPE that matches the relevant risk.

The following recommendations should be considered:

PPE compliant to the recommended EN/ISO standards should be selected.

- Eye/face protection 	: Wear safety glasses with side shields.

Standard EN 166 - Personal eye-protection - specifications.

- Skin protection

  - Hand protection 	: Wear safety gloves.

Wear working gloves when handling gas containers.

Standard EN 388 - Protective gloves against mechanical risk.

Standard EN 511 - Cold insulating gloves.

Wear leather safety gloves.

  - Other 	: Wear safety shoes while handling containers.

Standard EN ISO 20345 - Personal protective equipment - Safety footwear.

| • Respiratory protection | : Gas filters may be used if all surrounding conditions e.g. type and concentration of the contaminant(s) and duration of use are known. Use gas filters with full face mask, where exposure limits may be exceeded for a short-term period, e.g. connecting or disconnecting containers. Gas filters do not protect against oxygen deficiency. Standard EN 14387 - Gas filter(s), combined filter(s) and standard EN136, full face masks . |
| --- | --- |
| • Thermal hazards 8.2.3. Environmental exposure controls | : None in addition to the above sections. |
|  | Refer to local regulations for restriction of emissions to the atmosphere. See section 13 for specific methods for waste gas treatment. |

SECTION 9: Physical and chemical properties

## 9.1. Information on basic physical and chemical properties

| Appearance |  |
| --- | --- |
| - Physical state | : Refrigerated solidified gas |
| - Physical state at 20°C / 101.3kPa | : Gas |
| - Colour | : White. |
| Odour | : No odour warning properties. |
| Odour threshold | : Odour threshold is subjective and inadequate to warn of overexposure. |
| pH | : Not applicable for gases and gas mixtures. |
| Melting point / Freezing point | : -78.5 °C At atmospheric pressure dry ice sublimes into gaseous carbon dioxide. |
| Boiling point | : -56.6 °C |
| Flash point | : Not applicable. |
| Evaporation rate | : Not applicable. |
| Flammability (solid, gas) | : Non flammable. |
| Explosive limits | : Non flammable. |
| Vapour pressure [20°C] | : 57.3 bar(a) |
| Vapour pressure [50°C] | : Not applicable. |
| Vapour density | : Not applicable. |
| Relative density, liquid (water=1) | : 1.03 |
| Relative density, gas (air=1) | : 1.52 |
| Water solubility | : 2000 mg/l |
| Partition coefficient n-octanol/water (Log Kow) | : 0.83 |
| Auto-ignition temperature | : Non flammable. |
| Decomposition temperature | : Not applicable. |
| Viscosity | : No reliable data available. |
| Explosive properties | : Not applicable. |
| Oxidising properties 9.2. Other information | : Not applicable. |
| Molar mass | : 44 g/mol |
| Critical temperature [°C] | : 30 °C |
| Other data | : Gas/vapour heavier than air. May accumulate in confined spaces, particularly at or below ground level. |

SECTION 10: Stability and reactivity

| 10.1. Reactivity |  |
| --- | --- |
| 10.2. Chemical stability | No reactivity hazard other than the effects described in sub-sections below. |
| 10.3. Possibility of hazardous reactions | Stable under normal conditions. |
|  | None. |

## 10.4. Conditions to avoid

| 10.5. Incompatible materials |  |  | Avoid moisture in installation systems. |
| --- | --- | --- | --- |
| 10.6. Hazardous decomposition products |  |  | For additional information on compatibility refer to ISO 11114. |
|  |  |  | Under normal conditions of storage and use, hazardous decomposition products should not be produced. |
| SECTION 11: Toxicological information |  |  |  |
| 11.1. Information on toxicological effects Acute toxicity | : Unlike simple asphyxiants, carbon dioxide has the ability to cause death even when normal oxygen levels (20-21%) are maintained. 5% CO2 has been found to act synergistically to increase the toxicity of certain other gases (CO, NO2). CO2 has been shown to enhance the production of carboxy- or met-hemoglobin by these gases possibly due to carbon dioxide’s stimulatory effects on the respiratory and circulatory systems. For more information, see 'EIGA Safety Info 24: Carbon Dioxide, Physiological Hazards' at www.eiga.eu. |  |  |
| Skin corrosion/irritation | : No known effects from this product. |  |  |
| Serious eye damage/irritation | : No known effects from this product. |  |  |
| Respiratory or skin sensitisation | : No known effects from this product. |  |  |
| Germ cell mutagenicity | : No known effects from this product. |  |  |
| Carcinogenicity | : No known effects from this product. |  |  |
| Toxic for reproduction : Fertility | : No known effects from this product. |  |  |
| Toxic for reproduction : unborn child | : No known effects from this product. |  |  |
| STOT-single exposure | : No known effects from this product. |  |  |
| STOT-repeated exposure | : No known effects from this product. |  |  |
| Aspiration hazard | : Not applicable for gases and gas mixtures. |  |  |
| SECTION 12: Ecological information |  |  |  |

## 12.1. Toxicity

| Assessment | : No ecological damage caused by this product. |
| --- | --- |
| EC50 48h - Daphnia magna [mg/l] | : No data available. |
| EC50 72h - Algae [mg/l] | : No data available. |
| LC50 96 h - Fish [mg/l] 12.2. Persistence and degradability | : No data available. |
| Assessment | : No ecological damage caused by this product. |

## 12.3. Bioaccumulative potential

Assessment 	: No ecological damage caused by this product.

Not expected to bioaccumulate due to the low log Kow (log Kow < 4).

Refer to section 9.

12.4. Mobility in soil

Assessment 	: No ecological damage caused by this product.

12.5. Results of PBT and vPvB assessment

Assessment 	: Not classified as PBT or vPvB.

## 12.6. Other adverse effects

Other adverse effects 	: No known effects from this product.

| Effect on the ozone layer |  | : No effect on the ozone layer. |  |
| --- | --- | --- | --- |
| Global warming potential [CO2=1] |  | : 1 |  |
| Effect on global warming |  | : When discharged in large quantities may contribute to the greenhouse effect. Contains greenhouse gas(es). |  |
| SECTION 13: Disposal considerations |  |  |  |
| 13.1. Waste treatment methods |  | Contact supplier if guidance is required. Discharge to atmosphere in large quantities should be avoided. Do not discharge into any place where its accumulation could be dangerous. Ensure that the emission levels from local regulations or operating permits are not exceeded. Refer to the EIGA code of practice Doc.30 "Disposal of Gases", downloadable at http://www.eiga.org for more guidance on suitable disposal methods. Return unused product in original container to supplier. |  |
| List of hazardous waste codes (from Commission Decision 2000/532/EC as amended) |  | : 16 05 05 : Gases in pressure containers other than those mentioned in 16 05 04. |  |
| 13.2. Additional information |  | External treatment and disposal of waste should comply with applicable local and/or national regulations. |  |
| SECTION 14: Transport information |  |  |  |

## 14.1. UN number

| In accordance with ADR / RID / IMDG / IATA / ADN |  |
| --- | --- |
| UN-No. 14.2. UN proper shipping name | : 1845 |
| Transport by road/rail (ADR/RID) | : Not subject to ADR except for section 5.5.3. |
| Transport by air (ICAO-TI / IATA-DGR) | : Carbon dioxide, solid |
| Transport by sea (IMDG) 14.3. Transport hazard class(es) | : CARBON DIOXIDE, SOLID (DRY ICE) |
| Labelling | : |
| Transport by air (ICAO-TI / IATA-DGR) |  |
| Class / Div. (Sub. risk(s)) | : 9 |
| Transport by sea (IMDG) |  |
| Class / Div. (Sub. risk(s)) | : 9 |
| Emergency Schedule (EmS) - Fire | : F-C |
| Emergency Schedule (EmS) - Spillage 14.4. Packing group | : S-V |
| Transport by road/rail (ADR/RID) | : Not applicable |
| Transport by air (ICAO-TI / IATA-DGR) | : Not applicable |
| Transport by sea (IMDG) 14.5. Environmental hazards | : Not applicable |
| Transport by road/rail (ADR/RID) | : None. |
| Transport by air (ICAO-TI / IATA-DGR) | : None. |
| Transport by sea (IMDG) 14.6. Special precautions for user | : None. |
| Packing Instruction(s) |  |
| Transport by air (ICAO-TI / IATA-DGR) |  |
| Passenger and Cargo Aircraft | : 954. |
| Cargo Aircraft only | : 954. |  |
| Transport by sea (IMDG) | : P003 |  |
| Special transport precautions | : Avoid transport on vehicles where the load space is not separated from the driver's compartment. Ensure vehicle driver is aware of the potential hazards of the load and knows what to do in the event of an accident or an emergency. Before transporting product containers: - Ensure there is adequate ventilation. - Ensure that containers are firmly secured. - Ensure valve is closed and not leaking. - Ensure valve outlet cap nut or plug (where provided) is correctly fitted. - Ensure valve protection device (where provided) is correctly fitted. |  |

14.7. Transport in bulk according to Annex II of Marpol and the IBC Code

Not applicable.

# SECTION 15: Regulatory information

## 15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

EU-Regulations

| Restrictions on use | : None. |
| --- | --- |
| Seveso Directive : 2012/18/EU (Seveso III) National regulations | : Not covered. |
| Water hazard class (WGK) | : nwg - Non-hazardous to water |
| Kenn-Nr. | : 256 |
| Regulatory reference 15.2. Chemical safety assessment | : Ensure all national/local regulations are observed. |
|  | A CSA does not need to be carried out for this product. |

# SECTION 16: Other information

| Indication of changes | : Revised safety data sheet in accordance with commission regulation (EU) No 453/2010. |
| --- | --- |

| Abbreviations and acronyms | : ATE - Acute Toxicity Estimate CLP - Classification Labelling Packaging Regulation; Regulation (EC) No 1272/2008 REACH - Registration, Evaluation, Authorisation and Restriction of Chemicals Regulation (EC) No 1907/2006 EINECS - European Inventory of Existing Commercial Chemical Substances CAS# - Chemical Abstract Service number PPE - Personal Protection Equipment LC50 - Lethal Concentration to 50 % of a test population RMM - Risk Management Measures PBT - Persistent, Bioaccumulative and Toxic vPvB - Very Persistent and Very Bioaccumulative STOT- SE : Specific Target Organ Toxicity - Single Exposure CSA - Chemical Safety Assessment EN - European Standard UN - United Nations ADR - European Agreement concerning the International Carriage of Dangerous Goods by Road IATA - International Air Transport Association IMDG code - International Maritime Dangerous Goods RID - Regulations concerning the International Carriage of Dangerous Goods by Rail WGK - Water Hazard Class STOT - RE : Specific Target Organ Toxicity - Repeated Exposure UFI : Unique Formula Identifier |
| --- | --- |
| Training advice | : None. |
| Further information | : Classification in accordance with the procedures and calculation methods of Regulation (EC) 1272/2008 (CLP). Key literature references and sources of data are maintained in EIGA doc 169 : 'Classification and Labelling Guide', downloadable at http://www.Eiga.eu . |
| DISCLAIMER OF LIABILITY | : Before using this product in any new process or experiment, a thorough material compatibility and safety study should be carried out. Details given in this document are believed to be correct at the time of going to press. Whilst proper care has been taken in the preparation of this document, no liability for injury or damage resulting from its use can be accepted. End of document |